Acceptance protocol for SM100

Overview

| Acceptance test no. |  |
| --- | --- |
| Date | 01.01.2023 |
| Carried out by: |  |

Machine description and general picture

| Manufacturer |  |
| --- | --- |
| Machine / type |  |
| System / system part |  |
| Name of axis |  |
| Controller type | E94A |
| Serial number of controller (C00204/7) |  |
| Safety module type | SM100 |
| Serial number of safety module (C00204/6) |  |
| Circuit diagram version / relevant drawing sheets |  |
| Final customer |  |
| General picture of the machine |  |
| General picture of the safety circuit |  |

Function check (positive testing)

Description

Carry out the function check separately for every single drive.

Execution

| First commissioning | Please tick |  |
| --- | --- | --- |
| Recommissioning (e.g. after disassembly for machine transport) |  |  |
| Serial commissioning |  |  |

"Safe Torque Off" (STO) function

Function check:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status: | - Display of device status "Device is switched on" - Display of error status "No error" - STO deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate STO function | - Motor coasts down or is decelerated and held by a mechanical brake. - Display of device status "Safe torque off active" - Display of error status "No error" - No error concerning safety engineering |  |
| 4 | Deactivate STO | - No error concerning safety engineering - Display of device status "Safe torque off active" has disappeared - Display of error status "No error" |  |
| 5 | Traverse drive (observe C00142) | Setpoint/actual value comparison |  |

Error scenarios (validation through test according to EN ISO 13849-2)

(negative testing)

The below-described error scenarios exclusively refer to the functionality of the SM100. For further components contained in the safety structure, error scenarios must be checked as well.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Open circuit in first disconnecting path: Only activate disconnecting path SIA (if necessary, remove connecting wire) | - Motor coasts down or is decelerated and held by a mechanical brake. - Display of device status "Safe torque off active" - Display of error status "No error" |  |
| 2 | Restore disconnecting path | - Motor restarts (observe C00142) |  |
| 3 | Open circuit in second disconnecting path: Only activate disconnecting path SIB (if necessary, remove connecting wire) | - Motor coasts down or is decelerated and held by a mechanical brake. - Display of device status "Safe torque off active" - Display of error status "No error" |  |
| 4 | Restore disconnecting path | - Motor restarts (observe C00142) |  |
| When using the digital output DO1 on the SM100 for the plausibility check (only required, if error eliminations for protected cables (according to DIN EN ISO 13849-2:2013-02 and DIN EN 61800-5-2:2017-11) are not possible). |  |  |  |
| 5 | Only activate disconnecting path SIA (if necessary, remove connecting wire) | Digital output outputs "LOW level" |  |
| 6 | Activate both disconnecting paths SIA and SIB (if necessary, remove connecting wires) | Digital output outputs "HIGH level" |  |
| 7 | Restore functionality of disconnecting path SIA | Digital output outputs "LOW level" |  |
| 8 | Restore functionality of disconnecting path SIB | Digital output outputs "LOW level" |  |
| 9 | Stuck at High on disconnecting path B: Connect wire jumper, activate STO (safety switching device or an active sensor with OSSD outputs will change to error status. Remedy: Disconnect X80/SIB from safety switching device and only connect to 24VDC) | - Controller is disconnected via a pre-connected safety switching device or an active sensor - Motor coasts down or is decelerated and held by a mechanical brake. - Display of device status "Safe torque off active" - Display of error status "No error" |  |
| 10 | Reset sensor, acknowledge restart inhibit | Safety switching device and controller must remain inhibited |  |
| 11 | Remove wire jumper (if necessary, disconnect X80/SIB from 24VDC and re-connect to safety switching device), acknowledge restart inhibit | Safety switching device and controller are enabled (observe C00142) |  |

Protocol conclusion

Safety-oriented parameters			Check sum

not available						not available

Data back-up - SM100

|  | Storage medium |  |  | Storage location |
| --- | --- | --- | --- | --- |
|  | Type | Designation / version | Date |  |
| Circuit diagrams |  |  |  |  |

Countersignatures

The 'commissioning engineer'

hereby confirms that the above-described tests and checks were carried out properly by skilled personnel.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |

The 'machine manufacturer'

hereby confirms the correctness of the above-described parameterisation. The responsibility for the safety of the machine remains with the machine manufacturer.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |