Selecting the safety parameters for the SM301 V1.4

Issued by:			.....

Machine / type:		.....

Systems / system part:	.....

Name of axis:			.....

| Password |  |  |
| --- | --- | --- |
| 0.1 Password (min. 6 characters) | Standard „Lenze SM301“ | Own ..... |

| General |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 Module ID |  |  |  | 1.2 Safety address |  |  |  |  |  |  |
| 1.3 Motor encoder system | No motor encoder |  |  | Sin-cos encoder |  |  |  |  | Resolver |  |
| 1.4 Motor mounting direction | Motor rotating CW |  |  |  |  | Motor rotating CCW |  |  |  |  |
| 1.5 Sin-cos encoder PPR |  |  | 1.6 Number of resolver pole pairs |  |  |  |  |  |  |  |
| 1.7 Position encoder system | No position encoder |  |  |  | Analog encoder |  |  | Digital encoder |  |  |
| 1.8 Position encoder gearbox factor |  |  |  |  |  |  | % |  |  |  |
| 1.9 Pos. encoder mounting direction |  | As motor encoder |  |  |  |  | Inverted to motor encoder |  |  |  |

| SD-In1 (Safe input 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1.1 Sensor type | Input deactivated |  |  | Passive sensor |  | Active sensor |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.1.2 Sensor function | Free assignment |  |  |  |  |  |  |
|  | Emergency stop |  | Operating mode selector |  |  | Enable switch |  |
| 2.1.3 Free assignment | No function |  | STO |  | SS1 | SS2 |  |
|  | SLS1 |  | SLS2 |  | SLS3 | SLS4 |  |
|  | SDIpos |  | SDIneg |  |  |  |  |
| 2.1.4 Discrepancy time |  | msec | 2.1.5 Input delay |  |  |  | msec |

| SD-In2 (Safe input 2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.2.1 Sensor type | Input deactivated |  |  | Passive sensor |  | Active sensor |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.2.2 Sensor function | Free assignment |  |  |  |  |  |  |
|  | Emergency stop |  | Operating mode selector |  |  | Enable switch |  |
| 2.2.3 Free assignment | No function |  | STO |  | SS1 | SS2 |  |
|  | SLS1 |  | SLS2 |  | SLS3 | SLS4 |  |
|  | SDIpos |  | SDIneg |  |  |  |  |
| 2.2.4 Discrepancy time |  | msec | 2.2.5 Input delay |  |  |  | msec |

| SD-In3 (Safe input 3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.3.1 Sensor type | Input deactivated |  |  | Passive sensor |  | Active sensor |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.3.2 Sensor function | Free assignment |  |  |  |  |  |  |
|  | Emergency stop |  | Operating mode selector |  |  | Enable switch |  |
| 2.3.3 Free assignment | No function |  | STO |  | SS1 | SS2 |  |
|  | SLS1 |  | SLS2 |  | SLS3 | SLS4 |  |
|  | SDIpos |  | SDIneg |  |  |  |  |
| 2.3.4 Discrepancy time |  | msec | 2.3.5 Input delay |  |  |  | msec |

| SD-In4 (Safe input 4) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2.4.1 Sensor type | Input deactivated |  | Passive sensor |  | Active sensor |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |
| 2.4.2 Sensor function | Free assignment |  |  |  |  |  |
|  | Emergency stop |  | Operating mode selector |  | Enable switch |  |
| 2.4.3 Free assignment | No function |  | STO | SS1 | SS2 |  |
|  | SLS1 |  | SLS2 | SLS3 | SLS4 |  |
|  | SDIpos |  | SDIneg |  |  |  |
| 2.4.4 Discrepancy time |  | msec | 2.4.5 Input delay |  |  | msec |

| SD-Out1 (Safe Output 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.1 STO active | not evaluated | positive logic |  | negative logic |
| 3.2 SS1 active | not evaluated | positive logic |  | negative logic |
| 3.3 SS2 active | not evaluated | positive logic |  | negative logic |
| 3.4 SLS1 active | not evaluated | positive logic |  | negative logic |
| 3.5 SLS2 active | not evaluated | positive logic |  | negative logic |
| 3.6 SLS3 active | not evaluated | positive logic |  | negative logic |
| 3.7 SLS4 active | not evaluated | positive logic |  | negative logic |
| 3.8 SDIpos active | not evaluated | positive logic |  | negative logic |
| 3.9 SDIneg active | not evaluated | positive logic |  | negative logic |
| 3.10 ES active | not evaluated | positive logic |  | negative logic |
| 3.11 SLI active | not evaluated | positive logic |  | negative logic |
| 3.12 OMS | not evaluated | positive logic |  | negative logic |
| 3.13 SOS observed | not evaluated | positive logic |  | negative logic |
| 3.14 SLS1 observed | not evaluated | positive logic |  | negative logic |
| 3.15 SLS2 observed | not evaluated | positive logic |  | negative logic |
| 3.16 SLS3 observed | not evaluated | positive logic |  | negative logic |
| 3.17 SLS4 observed | not evaluated | positive logic |  | negative logic |
| 3.18 SDIpos observed | not evaluated | positive logic |  | negative logic |
| 3.19 SDIneg observed | not evaluated | positive logic |  | negative logic |
| 3.20 SSE active | not evaluated | positive logic |  | negative logic |
| 3.21 SD-In1 active | not evaluated | positive logic |  | negative logic |
| 3.22 SD-In2 active | not evaluated | positive logic |  | negative logic |
| 3.23 SD-In3 active | not evaluated | positive logic |  | negative logic |
| 3.24 SD-In4 active | not evaluated | positive logic |  | negative logic |
| 3.25 OMS active | not evaluated | positive logic |  | negative logic |
| 3.26 Error active | not evaluated | positive logic |  | negative logic |
| 3.27 Logic function | OR |  | AND |  |

| Safe Stopp (STO, SS1, SS2 and SOS) |  |  |
| --- | --- | --- |
| 4.1.1 Emergency stop function | STO | SS1 |
| 4.1.2 Restart – STO, SS1 | Acknowledged restart | Automatic restart |
| No entries required when functions SS1 or SS2 are not used |  |  |
| 4.1.3 Restart – SS2, SOS | Acknowledged restart | Automatic restart |
| 4.1.4 SS1, SS2 stop time |  | msec |
| 4.1.5 SS1 mode | STO after stop time | STO when n=0 |
| 4.1.6 SS1: Time lag from n=0 to STO |  | msec |
| 4.1.7 SS2 mode | SOS after stop time | SOS when n=0 |
| 4.1.8 SS1/SS2: Ramp monitoring | No ramp monitoring | Ramp monitoring active |
| 4.1.9 SS1, SS2: S-ramp time |  | % |
| 4.1.10 SS1, SS2: Start-offset ramp |  | % |
| 4.1.11 Speed window (n=0) |  | rpm |
| 4.1.12 SOS: Tolerance wndw (delta p=0) |  | incr. |

| Safe speed (SMS, SLS1-4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.3 SMS: Maximum speed Nmax |  |  |  | rpm |  |  |  |  |  |
| No entries required when "No motor encoder" is selected |  |  |  |  |  |  |  |  |  |
| 4.2.1 Response time speed acquisition | 12msec |  |  | 50msec |  |  | 100msec |  |  |
| 4.2.2 Speed comparison tolerance |  |  |  | rpm |  |  |  |  |  |
| 4.2.4 SMS: Reaction (n>Nmax) | STO |  |  | SS1 |  | SS2 |  |  |  |
| 4.2.5 SLS1: Limited speed Nlim1 |  | rpm |  | 4.2.6 SLS1: Braking time Nlim1 |  |  |  |  | msec |
| 4.2.7 SLS1: Reaction (n>Nlim1) | STO |  |  | SS1 |  | SS2 |  |  |  |
| 4.2.8 SLS1: Permitted direction | Both directions released |  |  | Positive direction released |  | Negative direction released |  |  |  |
| 4.2.9 SLS2: Limited speed Nlim2 |  | rpm |  | 4.2.10 SLS2: Braking time Nlim2 |  |  |  | msec |  |
| 4.2.11 SLS2: Reaction (n>Nlim2) | STO |  |  | SS1 |  | SS2 |  |  |  |
| 4.2.12 SLS2: Permitted direction | Both directions released |  |  | Positive direction released |  | Negative direction released |  |  |  |
| 4.2.13 SLS3: Limited speed Nlim3 |  | rpm |  | 4.2.14 SLS3: Braking time Nlim3 |  |  |  | msec |  |
| 4.2.15 SLS3: Reaction (n>Nlim3) | STO |  |  | SS1 |  | SS2 |  |  |  |
| 4.2.16 SLS3: Permitted direction | Both directions released |  |  | Positive direction released |  | Negative direction released |  |  |  |
| 4.2.17 SLS4: Limited speed Nlim4 |  | rpm |  | 4.2.18 SLS4: Braking time Nlim4 |  |  |  | msec |  |
| 4.2.19 SLS4: Reaction (n>Nlim4) | STO |  |  | SS1 |  | SS2 |  |  |  |
| 4.2.20 SLS4: Permitted direction | Both directions released |  |  | Positive direction released |  | Negative direction released |  |  |  |
| No entry required when only "STO" is selected as reaction of SMS and SLS |  |  |  |  |  |  |  |  |  |
| 4.2.21 SLS, SMS: Max. response time |  |  |  |  | msec |  |  |  |  |

| Operating mode selection (STM, SLI) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No entries required when function is not used |  |  |  |  |
| 4.3.1 OMS: Stop function | STO | SS1 |  | SS2 |
| 4.3.2 OMS: Motion function | Free traversing |  |  |  |
|  | SLS1 | SLS2 | SLS3 | SLS4 |
| 4.3.3 OMS: Function at low level | Std. operation |  | Spcl. operation |  |
| 4.3.4 SLI: Safely limited increment |  |  | Incr. |  |

| Cascading |  |  |
| --- | --- | --- |
| 4.4 CAS: Cascading function | No cascading | Cascading using SD-In4 |

| Safe direction (SDI) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4.5.1 SDI: Std. operation monitoring | Both directions released |  |  |  |
|  | Positive direction released |  |  | Negative direction released |
| No entries required when “Both directions released” is selected |  |  |  |  |
| 4.5.2 SDI: Delay time |  |  |  | msec |
| 4.5.3 SDI: Tolerance threshold |  |  |  | incr. |
| 4.5.4 SDI: Error response |  | STO | SS1 | SS2 |

| Safety bus general |  |  |
| --- | --- | --- |
| 5.1 S-bus: Configuration | No safety bus |  |
|  | PROFIsafe / PROFIBUS | PROFIsafe / PROFINET |
| No entries required when "No safety bus" is selected |  |  |
| 5.2 S-bus: Filter control data: STO | Pass through | Block |
| 5.3 S-bus: Filter control data: SS1 | Pass through | Block |
| 5.4 S-bus: Filter control data: SS2 | Pass through | Block |
| 5.5 S-bus: Filter control data: SLS1 | Pass through | Block |
| 5.6 S-bus: Filter control data: SLS2 | Pass through | Block |
| 5.7 S-bus: Filter control data: SLS3 | Pass through | Block |
| 5.8 S-bus: Filter control data: SLS4 | Pass through | Block |
| 5.9 S-bus: Filter control data: SDIpos | Pass through | Block |
| 5.10 S-bus: Filter control data: SDIneg | Pass through | Block |
| 5.11 S-bus: Filter control data: ES | Pass through | Block |
| 5.12 S-bus: Filter control data: OMS | Pass through | Block |
| 5.13 S-bus: Filter control data: PS_AIS | Pass through | Block |
| 5.14 S-bus: Filter control data: PS_AIE | Pass through | Block |
| 5.15 S-bus: Filter control data: SSE | Pass through | Block |
| 5.16 S-bus: Filter control data: SD-Out1 | Pass through | Block |

(Pass through = function is operated via bus)