Acceptance protocol for motor replacement

This acceptance protocol is only designed for component replacement without change of the safety functionality.

General

| Date | 01.01.2023 |
| --- | --- |
| Carried out by |  |

Machine description

| Manufacturer |  |
| --- | --- |
| Machine / type / number |  |
| System / system part |  |
| Name of axis |  |

Execution

| Step | Description | Actual status / value |  |
| --- | --- | --- | --- |
| 1 | Switch off controller and safety component if necessary |  |  |
| 2 | Read encoder type from old motor nameplate |  |  |
| 3 | If necessary, read ratio of old gearbox from nameplate |  |  |
| 4 | - Disconnect motor, encoder, and, if necessary, brake and separate fan - Replace motor and, if necessary, gearbox - Re-connect motor, encoder, and, if necessary, brake and separate fan |  |  |
| 5 | Read encoder type from new motor nameplate |  |  |
| 6 | Compare the two type codes. |  |  |
| 7 | Are the encoder designations identical? (This is the safety-relevant check!) | Please tick | yes |
|  |  |  | no |
| 8 | If necessary, read ratio of new gearbox from nameplate |  |  |
| 9 | Compare the two ratios. |  |  |
| 10 | Are the two ratios identical? (This is the safety-relevant check!) | Please tick | yes |
|  |  |  | no |
| 11 | Switch on controller and safety component if necessary |  |  |
| 12 | For motor connection via terminal strip Does the motor rotate in the expected direction? | Please tick | yes |
|  |  |  | no |
| 13 | For encoder connection via terminal strip - Activate a safety function with speed or direction monitoring - Does the motor react as expected? | Please tick | yes |
|  |  |  | no |

Countersignature

The 'service engineer'

hereby confirms that the above-described tests and checks were carried out properly by skilled personnel.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |