Acceptance protocol for SM302

Fill in the acceptance protocol separately for every single drive controller with SM302 safety module V1.1.

# Overview

| Acceptance test no. |  |
| --- | --- |
| Date | 01.01.2024 |
| Carried out by: |  |

# Machine description and general picture

| Manufacturer |  |  |
| --- | --- | --- |
| Machine / type / number |  |  |
| System / system part |  |  |
| Name of axis |  |  |
| Controller type | E94 |  |
| Serial number of controller (C00204/7) |  |  |
| Safety module type | SM302 V1.1 |  |
| Serial number of safety module (C00204/6) |  |  |
| Safety- / PROFIsafe- /FSoE-address |  |  |
| Motor type |  |  |
| Motor encoder type |  |  |
| Load encoder type (for two-encoder concept) |  |  |
| Motor-encoder combination released by Lenze for safe speed monitoring | Please tick | yes |
|  |  | no |
| Gearbox factor between motor and load |  |  |
| Feed constant |  |  |
| Circuit diagram version / used drawing sheets |  |  |
| Final customer |  |  |
| General picture of the machine |  |  |
| General picture of the safety circuit |  |  |

# Function check (positive testing)

Description

Carry out the function check separately for every single motor (as far as permitted by the machine). Exception: safe cascading.

If possible, application error messages should be switched to "No response" for the check to ensure that only error messages of the safety module will be displayed.

Execution

| First commissioning | Please tick |  |
| --- | --- | --- |
| Recommissioning (e.g. after disassembly for machine transport) |  |  |
| Serial commissioning |  |  |

Use the parameters displayed in C15000–C15902 to check the selected parameters. Print out the selected parameters using the print function of the Engineer and attach the printout to the acceptance protocol.

Mark functions that are not used as "not used". To improve the overview, the corresponding table in this document can be deleted.

# "Safe torque off" function (SSE or free assignment / STO) P.1:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - STO deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate STO function | - Drive coasts down or is decelerated by a mechanical brake. - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate STO | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 5 | Traverse motor | Setpoint/actual value comparison |  |
| When using the function as an emergency stop, operation mode selection is used |  |  |  |
| 6 | Initial status | - Special operation mode activated - Enable switch actuated - Device status display "Device is switched on" - Error status display "No error" - STO deactivated |  |
| 7 | Traverse motor | Setpoint/actual value comparison |  |
| 8 | Activate STO function | - Drive coasts down or is decelerated by a mechanical brake. - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 9 | Deactivate STO | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 10 | Traverse motor | Setpoint/actual value comparison |  |

# "Safe stop 1" function (SSE or free assignment / SS1) P.2:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SS1 deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SS1 function | - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in SM302 - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SS1 | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 5 | Traverse motor | Setpoint/actual value comparison |  |
| When using the function as an emergency stop, operation mode selection is used |  |  |  |
| 6 | Initial status | - Special operation mode activated - Enable switch actuated - Device status display "Device is switched on" - Error status display "No error" - SS1 deactivated |  |
| 7 | Traverse motor | Setpoint/actual value comparison |  |
| 8 | Activate SS1 function | - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in SM302 - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 9 | Deactivate SS1 | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 10 | Traverse motor | Setpoint/actual value comparison |  |

# "Safe stop 2 / safe operating stop" function (free assignment / SS2 or special operation) P.3:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SS2 deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SS2 function | - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SS2 | - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor | Setpoint/actual value comparison |  |

# "Restart inhibit" function (STO / SS1 or SS2 / SOS) P.4:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | STO / SS1 activated |  |
| 2 | Reset activation signal | Setpoint/actual value comparison |  |
| 3 | If necessary, acknowledge restart inhibit | STO / SS1 deactivated |  |
| 4 | Initial status | SS2 / SOS activated |  |
| 5 | Reset activation signal | Setpoint/actual value comparison |  |
| 6 | If necessary, acknowledge restart inhibit | SS2 / SOS deactivated |  |

# "Safely-limited speed 1" function (free assignment / SLS1, SREF or special operation) P.5:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SLS1 deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SLS1 function | - Motor traverses along the ramp below the limit value - If necessary, device status display "Speed has been limited" (depending on the basic drive function used) - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS1 | - Motor accelerates again - No errors concerning safety engineering - Error status display "No error" |  |

# Safely-limited speed 2" function (free assignment / SLS2, SREF or special operation) P.6:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SLS2 deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SLS2 function | - Motor traverses along the ramp below the limit value - If necessary, device status display "Speed has been limited" (depending on the basic drive function used) - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS2 | - Motor accelerates again - No errors concerning safety engineering - Error status display "No error" |  |

# "Safely-limited speed 3" function (free assignment / SLS3, SREF or special operation) P.7:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SLS3 deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SLS3 function | - Motor traverses along the ramp below the limit value - If necessary, device status display "Speed has been limited" (depending on the basic drive function used) - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS3 | - Motor accelerates again - No errors concerning safety engineering - Error status display "No error" |  |

# "Safely-limited speed 4" function (free assignment / SLS4, SREF or special operation) P.8:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SLS4 deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SLS4 function | - Motor traverses along the ramp below the limit value - If necessary, device status display "Speed has been limited" (depending on the basic drive function used) - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS4 | - Motor accelerates again - No errors concerning safety engineering - Error status display "No error" |  |

# "Safe maximum speed" function P.9:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |

# "Safe cascading" function (SSE with STO or SS1) P.10:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SSE deactivated |  |
| 2 | Traverse motors | Setpoint/actual value comparison |  |
| 3 | Activate SSE function | If SS1 is used - Motors decelerate along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in SM302 or if STO is used - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Deactivate SSE, acknowledge restart | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 5 | Traverse motors | Setpoint/actual value comparison |  |

# "Safely-limited increment" function (SLI) P.11:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SLI deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate SLI function | - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Traverse motor | - Error status display "No error" - No errors concerning safety engineering - Motor stops automatically when reaching the permissible increment |  |
| 5 | Deactivate SLI | - Error status display "No error" - No errors concerning safety engineering |  |
| 6 | Traverse motor | Setpoint/actual value comparison |  |

# "Special operation" function (OMS with ES) P.12:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - OMS deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Activate special operation mode via OMS | If SSx is used as stop function - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in SM302 for SS1 or if STO is used - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Activate motion via ES and travel command | If SLSx is used as motion function - Motor traverses below the limit value - If necessary, device status display "Speed has been limited" (depending on the basic drive function used) - Error status display "No error" - No errors concerning safety engineering |  |
| 5 | Deactivate motion via ES | If SSx is used as stop function - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in SM302 for SS1 or if STO is used - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 6 | - Deactivate OMS - Acknowledge restart | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 7 | Traverse motor | Setpoint/actual value comparison |  |
| Use of SLI in special operation |  |  |  |
| 8 | Activate motion via ES and travel command | Motor stops automatically when reaching the permissible increment |  |

# "Repair mode" function (RMS with ES) P.13:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - RMS deactivated |  |
| 2 | Activate repair mode via RMS | Device status display "Device is switched on" |  |
| 3 | Reset error if necessary | - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Activate motion via ES and travel command | - Motor traverses - Error status display "No error" - No errors concerning safety engineering |  |
| 5 | Deactivate motion via ES | If SS1 is used as stop function - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in SM302 for SS1 or if STO is used - Device status display "Safe torque off active" - Error status display "No error" - No errors concerning safety engineering |  |
| 7 | - Deactivate RMS - Acknowledge restart | - No errors concerning safety engineering - Device status display "Safe torque off active" has disappeared - Error status display "No error" |  |
| 8 | Traverse motor | Setpoint/actual value comparison |  |

# "Safe direction" function (SDI) P.14:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - SDIpos and SDIneg deactivated |  |
| 2 | Traverse motor in positive direction | Setpoint/actual value comparison |  |
| 3 | Activate SDIneg function | - Motor decelerates along ramp to standstill - If necessary, error status display "Positive direction of rotation has been limited" (depends on basic drive function used) - If necessary, device status display |  |
| 4 | Deactivate SDIneg | - Motor accelerates again - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor in negative direction | Setpoint/actual value comparison |  |
| 6 | Activate SDIpos function | - Motor decelerates along ramp to standstill - If necessary, error status display "Negative direction of rotation has been limited" (depends on basic drive function used) - If necessary, device status display |  |
| 7 | Deactivate SDIpos | - Motor accelerates again - No errors concerning safety engineering - Error status display "No error" |  |

# "Safe homing" function (SREF) P.15:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position not available |  |
| 2 | Traverse motor in the direction of the SREF_Start-mark | Setpoint/actual value comparison |  |
| 3 | SREF_Start-mark reached resp. SREF_Start via safety-bus | - SREF_active is set - Depending of the 9400 homing mode the direction of motion can change into the direction of the Ref_Load-mark |  |
| 4 | SREF_Load-mark reached resp. SREF_Load via safety-bus | - SREF_active is reset - SREF_available is set |  |
| If the restart condition „load stored home position“ is used, after switching-on the mini-homing is necessary. |  |  |  |
| 5 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position not available - SREF_active is set |  |
| 6 | Traverse motor | Setpoint/actual value comparison |  |
| 7 | fourfold travelling distance of „tolerance position comparison“ is reached | - SREF_active is reset - SREF_available is set |  |

If the same edge is used for SREF_Start and SREF_Load, step 3 will be skipped and when the SREF_Start-mark is reached directly SREF_available is set.

# "Safe cam 1" function (SCA) P.16:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint/actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safe cam 2" function (SCA) P.17:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint/actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safe cam 3" function (SCA) P.18:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint/actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safe cam 4" function (SCA) P.19:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint/actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safely-limited position 1" function (SLP) P.20:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate SLP1 function | - No errors concerning safety engineering - Error status display "No error" |  |
| 3 | Traverse motor within the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |
| 4 | Deactivate SLP1 | - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor out of the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |

# "Safely-limited position 2" function (SLP) P.21:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate SLP2 function | - No errors concerning safety engineering - Error status display "No error" |  |
| 3 | Traverse motor within the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |
| 4 | Deactivate SLP2 | - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor out of the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |

# "Safely-limited position 3" function (SLP) P.22:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate SLP3 function | - No errors concerning safety engineering - Error status display "No error" |  |
| 3 | Traverse motor within the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |
| 4 | Deactivate SLP3 | - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor out of the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |

# "Safely-limited position 4" function (SLP) P.23:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate SLP4 function | - No errors concerning safety engineering - Error status display "No error" |  |
| 3 | Traverse motor within the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |
| 4 | Deactivate SLP4 | - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor out of the monitored area | - No errors concerning safety engineering - Error status display "No error" |  |

# "Position depending safe speed" function (PDSS) P.24:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - Home position available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate PDSS function (if not permanently active) | - No errors concerning safety engineering - Error status display "No error" |  |
| 3 | Traverse motor within the monitored area | - No errors concerning safety engineering - Error status display "No error" - Motor decelerates before the position limits are exceeded |  |
| If the SCS-areas are used |  |  |  |
| 4 | Traverse motor into the negative SCS-area | - No errors concerning safety engineering - Error status display "No error" |  |
| 5 | Traverse motor into the positive SCS-area | - No errors concerning safety engineering - Error status display "No error" |  |

# "Safe output SD-Out1" P.25:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Checking all states of the truth table | All switching states of the output correspond to the truth table. |  |
| Truth table |  |  |  |

# Error scenarios (validation through test according to EN ISO 13849-2)

(negative testing)

The below-described error scenarios exclusively refer to the functionality of the SM302. For further components contained in the safety chain, error scenarios must be checked as well.

At a series-commissioning with identical parameter sets the negative tests can be skipped from the 2nd axis on, because with these tests the correct input of the parameter values is checked. The sameness of the parameter sets can be verified by a comparison of the CRC’s.

# Safe input x (SD-In x) as Ref-In N.1

Used: 		Not used:

SD-In: 	Sensor type:

The following test only needs to be performed when using two separate sensors and sequential actuation of the sensors.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| When parameterising the safe input as "active sensor" or "passive sensor": |  |  |  |
| 1 | - Open circuit in channel A: (remove connecting wire) - move to the homing mark, so that SREF_Start would be set | SREF active is not set |  |
| 2 | Restore functionality of channel A | Error status display "No error" |  |
| 3 | - Open circuit in channel B: (remove connecting wire) - Set SREF_Start - move to the homing mark, so that SREF_Load would be set | - SREF active is reset after the timeout - SREF available is not set - Error status display "SREF Timeout" |  |
| 4 | Restore functionality of channel B | - Error acknowledgement possible - Error status display "No error" |  |
| Only when parameterising the safe input as "passive sensor": |  |  |  |
| 5 | Stuck at High (short circuit to 24VDC) on channel A: (connect wire jumper) | - SREF active is set, if rising edge is parameterized - Error status display "Stuck at High at SD-In x, channel A“latest after about 10s |  |
| 6 | - Restore functionality of channel A - Acknowledge error | - Error acknowledgement possible - Error status display "No error" |  |
| 7 | Stuck at High (short circuit to 24VDC) on channel B: (connect wire jumper) | - SREF active is reset - SREF available is set, if rising edge is parameterized - Error status display "Stuck at High at SD-In x, channel B“latest after about 10s |  |
| 8 | - Restore functionality of channel B - Acknowledge error | Error status display "No error" |  |
| 9 | Short circuit between the channels A and B (connect wire jumper) | - SREF active or SREF available is set - Error status display "Stuck at High at SD-In x, channel x“ latest after about 10s |  |
| 10 | - Restore functionality of channels A and B - Acknowledge error | - Error acknowledgement possible - Error status display "No error" |  |

# Safe input 1 (SD-In1) N.2

Used: 		Not used:

Sensor type: 	Sensor function: 	Free assignment:

The following test only has to be executed, if the input is parameterized as a passive sensor. If the input is parameterized as the homing input, the test needs not to be executed.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In1, channel x“ |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - Error status display "No error" |  |

# Safe input 2 (SD-In2) N.3

Used: 		Not used:

Sensor type: 	Sensor function: 	Free assignment:

The following test only has to be executed, if the input is parameterized as a passive sensor. If the input is parameterized as the homing input, the test needs not to be executed.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In2, channel x“ |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - Error status display "No error" |  |

# Safe input 3 (SD-In3) N.4

Used: 		Not used:

Sensor type: 	Sensor function: 	Free assignment:

The following test only has to be executed, if the input is parameterized as a passive sensor. If the input is parameterized as the homing input, the test needs not to be executed.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In3, channel x“ |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - Error status display "No error" |  |

# Safe input 4 (SD-In4) N.5

Used: 		Not used:

Sensor type: 	Sensor function: 	Free assignment:

The following test only has to be executed, if the input is parameterized as a passive sensor. If the input is parameterized as the homing input, the test needs not to be executed.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In4, channel x“ |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - Error status display "No error" |  |

# "Safe stop 1" function (SSE, free assignment/ SS1 or special operation mode) N.6

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select deceleration ramp in basic device longer (C00105 + C00106) than stopping time in SM302 - Activate SS1 | - Motor traverses along deceleration ramp - Device status display "Operation/quick stop active" Without ramp monitoring after expiry of the stopping time - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Safe torque off active" - Error status display "SS1/SS2: Stopping time exceeded" With ramp monitoring after exceeding the ramp - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Safe torque off active" - Error status display "SS1/SS2: Speed ramp exceeded" |  |
| 2 | - Reset deceleration ramp in basic device to the original values - Acknowledge error | - Error status display "No error" |  |

# "Safe stop 2" function (free assignment / SS2 or special operation mode) N.7

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select deceleration ramp in basic device longer (C00105 + C00106) than stopping time in SM302 - Activate SS2 | - Motor traverses along deceleration ramp - Device status display "Operation/quick stop active" Without ramp monitoring after expiry of the stopping time - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Safe torque off active" - Error status display "SOS: Tolerance limit exceeded" With ramp monitoring after exceeding the ramp - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Safe torque off active" - Error status display "SS1/SS2: Speed ramp exceeded" |  |
| 2 | - Reset error - Issue a motion command | - Motor traverses Exceeding the position tolerance - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Safe torque off active" - Error status display "SOS: Tolerance limit exceeded" |  |
| 3 | - Reset deceleration ramp in basic device to the original values - Acknowledge error | - Error status display "No error" |  |

# "Restart inhibit" function via AIS-terminal (STO / SS1 or SS2 / SOS) N.8

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| Acknowledged restart for STO / SS1 is used |  |  |  |
| 1 | Initial status STO / SS1 activated | Power output stage is inhibited |  |
| 2 | Reset activation signal | Power output stage remains inhibited |  |
| 3 | Activate acknowledgement input between 0.3s and 10s | - Power output stage is enabled - Motor can start |  |
| Acknowledged restart for SS2 / SOS is used |  |  |  |
| 4 | Initial status - SS2 / SOS activated | - Power output stage remains enabled - SOS is monitored |  |
| 5 | Reset activation signal | - Power output stage remains enabled - SOS continues to be monitored |  |
| 6 | Activate acknowledgement input between 0.3s and 10s | - Power output stage remains enabled - Motor can start |  |

# "Safely-limited speed 1" function (free assignment / SLS1, SREF or special operation) N.9

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim1 in SM302 - Activate SLS1 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS1: Nlim1 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Select deceleration ramp in basic device longer than the braking time in SM302 - Acknowledge error | Error status display "No error" |  |
| 3 | Activate SLS1 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS1: Nlim1 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | Error status display "No error" |  |

# "Safely-limited speed 2" function (free assignment / SLS2, SREF or special operation) N.10

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim2 in SM302 - Activate SLS2 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS2: Nlim2 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Select deceleration ramp in basic device longer than the braking time in SM302 - Acknowledge error | Error status display "No error" |  |
| 3 | Activate SLS2 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS2: Nlim2 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | - Error status display "No error" |  |

# "Safely-limited speed 3" function (free assignment / SLS3, SREF or special operation) N.11

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim3 in SM302 - Activate SLS3 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS3: Nlim3 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Select deceleration ramp in basic device longer than the braking time in SM302 - Acknowledge error | Error status display "No error" |  |
| 3 | Activate SLS3 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS3: Nlim3 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | - Error status display "No error" |  |

# "Safely-limited speed 4" function (free assignment / SLS4, SREF or special operation) N.12

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim4 in SM302 - Activate SLS4 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS4: Nlim4 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Select deceleration ramp in basic device longer than the braking time in SM302 - Acknowledge error | Error status display "No error" |  |
| 3 | Activate SLS4 | - Motor traverses along deceleration ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS4: Nlim4 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | Error status display "No error" |  |

# "Safe maximum speed" function N.13

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Select setpoint speed in basic device higher than SMS in SM302 | - Motor traverses to new setpoint - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SMS exceeded" |  |
| 2 | - Reset setpoint speed in basic device to original value - Acknowledge error | Error status display "No error" |  |

# "Safe direction" function	 N.14

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Set controller inhibit or activate STO | Controller is inhibited |  |
| 2 | - Activate SDIneg (positive direction is inhibited) - Move motor manually in positive direction (if necessary, release brake) | Error status display "SDIneg: Wrong direction" |  |
| 3 | Acknowledge error | Error status display "No error" |  |
| 4 | - Activate SDIpos (negative direction is inhibited) - Move motor manually in negative direction (if necessary, release brake) | Error status display "SDIpos: Wrong direction" |  |
| 5 | Acknowledge error | Error status display "No error" |  |

If the motor cannot be moved manually, traverse the motor applicatively in the inhibited direction. For this, it may be necessary to deactivate the limitation of direction in the FB editor (e.g. control of LS_Limiter).

# "Safely-limited increment" function in normal operation N.15

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Select the permissible number of increments in the basic device higher than SLI in SM302 | - Motor passes limit of permissible increments - Fault reaction STO is activated - Error status display "SLI: Safely limited increment exceeded" |  |
| 2 | - Reset the permissible number of increments in the basic device to the original value - Acknowledge error | Error status display "No error" |  |

# "Safely-limited increment" function in special operation N.16

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Select the permissible number of increments in the basic device higher than SLI in SM302 | - Motor passes limit of permissible increments - Stop function of the operating mode selection is activated - Error status display "SLI: Safely limited increment exceeded" |  |
| 2 | - Reset the permissible number of increments in the basic device to the original value - Acknowledge error | Error status display "No error" |  |

# "Safe homing" function N.17

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SREF is not available - SREF_Load-mark is manipulated, so that it can’t be detected |  |
| 2 | Traverse the motor to the mark, that SREF_available would be set | - Mark is not detected - SLS stays activated - SREF_available is not set - Timeout-message |  |

# "Safely-limited position 1" function N.18

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SREF is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP1 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP1: Position area exceeded“ |  |
| 4 | - Deactivate SLP1 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP1 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP1: Position area exceeded“ |  |
| 7 | - Deactivate SLP1 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Safely-limited position 2" function N.19

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SREF is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP2 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP2: Position area exceeded“ |  |
| 4 | - Deactivate SLP2 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP2 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP2: Position area exceeded“ |  |
| 7 | - Deactivate SLP2 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Safely-limited position 3" function N.20

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SREF is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP3 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP3: Position area exceeded“ |  |
| 4 | - Deactivate SLP3 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP3 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP3: Position area exceeded“ |  |
| 7 | - Deactivate SLP3 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Safely-limited position 4" function N.21

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SREF is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP4 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP4: Position area exceeded“ |  |
| 4 | - Deactivate SLP4 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP4 function | - Error status display "No error" - No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP4: Position area exceeded“ |  |
| 7 | - Deactivate SLP4 - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Position-dependent safe speed" function N.22

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SREF is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Traverse motor in positive direction | Error status display „PDSS: Speed limit exceeded“ when the curve is exceeded |  |
| 3 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 4 | Traverse motor in negative direction | Error status display „PDSS: Speed limit exceeded“ when the curve is exceeded |  |
| 5 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| If the SCS-areas are used |  |  |  |
| 6 | Traverse motor in positive direction with speed light above the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the SCS-area is reached |  |
| 7 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 8 | Traverse motor in positive direction with speed below the SCS-limit and then accelerate above the SCS-limit | Error status display „PDSS: Creeping speed SCS exceeded“ |  |
| 9 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 10 | Traverse motor in positive direction with speed below the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the SCS-area is exceeded |  |
| 11 | - Deactivate PDSS or activate the repair mode - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 12 | - Traverse motor back into the PDSS-area - Activate PDSS | - Error status display "No error" - No errors concerning safety engineering |  |
| 13 | Traverse motor in negative direction with speed light above the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the SCS-area is reached |  |
| 14 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 15 | Traverse motor in negative direction with speed below the SCS-limit and then accelerate above the SCS-limit | Error status display „PDSS: Creeping speed SCS exceeded“ |  |
| 16 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 17 | Traverse motor in negative direction with speed below the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the SCS-area is exceeded |  |
| 18 | - Deactivate PDSS or activate the repair mode - Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |
| 19 | - Traverse motor back into the PDSS-area - Activate PDSS | - Error status display "No error" - No errors concerning safety engineering |  |
| If the diagnostic marks are used |  |  |  |
| 20 | Simulation of a mark at a wrong position | - Mark is detected - Error “Diagnostic mark detected in invalid position” |  |
| 21 | Acknowledge error | - Error status display "No error" - No errors concerning safety engineering |  |

# "Safety-bus" N.23

Used: 		Not used:

If PROFIsafe or FSoE is used to activate safety functions in SM302, check whether the safety function with the highest priority will be activated if a bus error occurs. When a bus error occurs, the SM302 interprets all safety-bus bits passed through as FALSE. (FALSE activates the corresponding function)

| Priority | Function |
| --- | --- |
| high | STO SS1 SS2 SLSx, SDI, SLI, SLP, … |
| low |  |

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Device is switched on" - Error status display "No error" - STO deactivated |  |
| 2 | Traverse motor | Setpoint/actual value comparison |  |
| 3 | Trigger bus error | - Function with the highest priority is activated (Important: The non-safe communication will be interrupted, and the corresponding monitoring functions will be activated in the controller.) |  |
| 4 | - Reset bus error - Wait (up to 1 min. at PROFIsafe) | - No errors concerning safety engineering - Error status display "No error" |  |

# Two-encoder concept N.24

Used: 		Not used:

If a two-encoder concept is used to measure the safe speed and the safe position where required in SM302, check whether the SM302 detects discrepancies exceeding the tolerances.

You have to activate phase control, if necessary. You have to manipulate the position encoder such that a speed and position discrepancy can occur (e.g. by covering the laser or removing the encoder). Then the drive can be operated without position evaluation of the position encoder. The encoder will still be evaluated by the SM302.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Traverse motor quickly (speed-tolerance threshold must be exceeded) | - Motor traverses beyond tolerance limit - STO is activated - Error status display "SSM: Speed comparison error" |  |
| 2 | - Activate position control, if necessary - Restore evaluation of the two encoders - Acknowledge error | - No errors concerning safety engineering - Error status display "No error" |  |
| Only used if functions with absolute position monitoring are used |  |  |  |
| 3 | Traverse motor slowly (position-tolerance threshold must be exceeded) | - Motor traverses beyond tolerance limit - STO is activated - Error status display „Tolerance position comparison exceeded“ |  |
| 4 | - Activate position control, if necessary - Restore evaluation of the two encoders - Acknowledge error | - No errors concerning safety engineering - Error status display "No error" |  |

maybe it only can be tested, if the slip compensation is deactivated

# Protocol conclusion

Safety-oriented parameters			Check sum (CRC)

File name:				.bin		0x

Data back-up SM302

|  | Storage medium |  |  | Storage location |
| --- | --- | --- | --- | --- |
|  | Type | Designation / version | Date |  |
| Parameters |  |  |  |  |
| Circuit diagrams |  |  |  |  |

Countersignatures

The 'commissioning engineer'

hereby confirms that the above-described tests and checks were carried out properly by skilled personnel.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |

The 'machine manufacturer'

hereby confirms that the printout of the parameterisation carried out (attachment of this protocol) will be compared for compliance with the manufacturer's own specification. The responsibility for the safety of the machine remains with the machine manufacturer.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |