Acceptance protocol for Extended Safety

Fill in the acceptance protocol separately for every single drive controller with Extended Safety V1.5.

# Overview

| Acceptance test no. |  |
| --- | --- |
| Date | 01.01.2026 |
| Carried out by: |  |

# Machine description and general picture

| Manufacturer |  |  |
| --- | --- | --- |
| Machine / type / number |  |  |
| System / system part |  |  |
| Name of axis |  |  |
| Drive controller type (0x2000:001) |  |  |
| Serial number of drive controller (0x2000:002) |  |  |
| Version parameter set (0x2114:001) | i950 ES V1.5 |  |
| Serial number of safety component (0x212D:003) |  |  |
| Safety- / PROFIsafe- / FSoE-address |  |  |
| Motor type (0x2C01:010) |  |  |
| Motor encoder type |  |  |
| Load encoder type (for two-encoder concept) |  |  |
| Motor-encoder combination released by Lenze for safe speed monitoring | Please tick | yes |
|  |  | no |
| Gearbox factor between motor and load |  |  |
| Feed constant |  |  |
| Circuit diagram version / relevant drawing sheets |  |  |
| Final customer |  |  |
| General picture of the machine |  |  |
| General picture of the safety circuit |  |  |

# Function check (positive testing)

Description

Carry out the function check separately for every single motor (as far as permitted by the machine). Exception: safe cascading.

If possible, application error messages should be switched to "No response" for the check to ensure that only error messages of the safety component will be displayed.

Execution

| First commissioning | Please tick |  |
| --- | --- | --- |
| Recommissioning (e.g. after disassembly for machine transport) |  |  |
| Serial commissioning |  |  |
| Function change |  |  |

Use the parameters displayed in 0x2114-28E9 and 0xF980 to check the selected parameters. Print out the selected parameters using the print function of the PLC Designer or EASY Starter and attach the printout to the acceptance protocol.

Mark functions that are not used as "Not used". Then the corresponding table can be deleted.

# "Safe torque off" function (SSE or STO) P.1:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - STO deactivated - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate STO function | - Drive coasts down or is decelerated by a mechanical brake. - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 4 | Deactivate STO | - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 5 | Traverse motor | Setpoint / actual value comparison |  |
| When using the function as an emergency stop, operation mode selection is used |  |  |  |
| 6 | Initial status | - Special operation / repair mode activated - Enable switch actuated - STO deactivated - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 7 | Traverse motor | Setpoint / actual value comparison |  |
| 8 | Activate STO function | - Drive coasts down or is decelerated by a mechanical brake. - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 9 | Deactivate STO | - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 10 | Traverse motor | Setpoint / actual value comparison |  |

# "Safe stop 1" function (SSE or SS1) P.2:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SS1 deactivated - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SS1 function | - Motor decelerates along QSP application-ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES - Drive switches to STO - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 4 | Deactivate SS1 | - Device status display "Operation enabled” - Safe torque off-display not active - No errors concerning safety engineering |  |
| 5 | Traverse motor | Setpoint / actual value comparison |  |
| When using the function as an emergency stop, operation mode selection is used |  |  |  |
| 6 | Initial status | - Special operation / repair mode activated - Enable switch actuated - SS1 deactivated - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 7 | Traverse motor | Setpoint / actual value comparison |  |
| 8 | Activate SS1 function | - Motor decelerates along QSP application-ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES - Drive switches to STO - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 9 | Deactivate SS1 | - Device status display "Operation enabled” - Safe torque off-display not active - No errors concerning safety engineering |  |
| 10 | Traverse motor | Setpoint / actual value comparison |  |

# "Safe stop 2" function (SS2 or special operation) P.3:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SS2 deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SS2 function | - Motor decelerates along QSP ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES - Drive switches to SOS - No errors concerning safety engineering |  |

# "Safe operating stop" function (SS2, SOS or special operation) P.4:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SOS deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SOS function | - Device status display "Operation/quick stop active" - No errors concerning safety engineering |  |
| 4 | Deactivate SOS | - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 5 | Traverse motor | Setpoint / actual value comparison |  |

# "Restart inhibit" function (STO or SOS) P.5:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | STO activated |  |
| 2 | Reset activation signal | Setpoint / actual value comparison |  |
| 3 | Acknowledge restart inhibit | STO deactivated |  |
| 4 | Initial status | SOS activated |  |
| 5 | Reset activation signal | Setpoint / actual value comparison |  |
| 6 | Acknowledge restart inhibit | SOS deactivated |  |

# "Safely-limited speed 1" function (SLS1, SHom or special operation) P.6:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SLS1 deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SLS1 function | - Motor decelerates along the ramp below the limit value - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS1 | - Motor accelerates again - Device status display "Operation enabled" - No errors concerning safety engineering |  |

# "Safely-limited speed 2" function (SLS2, SHom or special operation) P.7:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SLS2 deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SLS2 function | - Motor decelerates along the ramp below the limit value - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS2 | - Motor accelerates again - Device status display "Operation enabled" - No errors concerning safety engineering |  |

# "Safely-limited speed 3" function (SLS3, SHom or special operation) P.8:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SLS3 deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SLS3 function | - Motor decelerates along the ramp below the limit value - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS3 | - Motor accelerates again - Device status display "Operation enabled" - No errors concerning safety engineering |  |

# "Safely-limited speed 4" function (SLS4, SHom or special operation) P.9:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SLS4 deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SLS4 function | - Motor decelerates along the ramp below the limit value - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 4 | Deactivate SLS4 | - Motor accelerates again - Device status display "Operation enabled" - No errors concerning safety engineering |  |

# "Safe maximum speed" function P.10:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |

# "Safe speed monitor" function (SSM) P.11:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor faster than the monitored speed | Safe output switches or bit on the s-bus switches off |  |
| 3 | Traverse motor slower than the monitored speed | Safe output switches or bit on the s-bus switches on |  |

# "Safe cascading" function (SSE with STO or SS1) P.12:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SSE deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motors | Setpoint / actual value comparison |  |
| 3 | Activate SSE function | If SS1 is used - Motors decelerate along QSP application-ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES or if STO is used - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 4 | Deactivate SSE, acknowledge restart | - Device status display "Operation enabled” - Safe torque off-display not active - No errors concerning safety engineering |  |
| 5 | Traverse motors | Setpoint / actual value comparison |  |

# "Safely-limited increment" function (SLI) P.13:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SLI deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SLI function | No errors concerning safety engineering |  |
| 4 | Traverse motor | - Motor stops automatically when reaching the permissible increment - No errors concerning safety engineering |  |
| 5 | Deactivate SLI | No errors concerning safety engineering |  |
| 6 | Traverse motor | Setpoint / actual value comparison |  |

# "Special operation" function (OMS with ES) P.14:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - OMS deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate special operation mode via OMS | If SSx is used as stop function - Motor decelerates along QSP application-ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES for SS1 or if STO is used - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 4 | Activate motion via ES and travel command | - Device status display "Operation enabled" - No errors concerning safety engineering If SLSx is used as motion function - Motor traverses below the limit value |  |
| 5 | Deactivate motion via ES | If SSx is used as stop function - Motor decelerates along QSP application-ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES for SS1 or if STO is used - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 6 | - Deactivate OMS - Acknowledge restart if necessary | - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 7 | Traverse motor | Setpoint / actual value comparison |  |
| Use of SLI in special operation |  |  |  |
| 8 | Activate motion via ES and travel command | Motor stops automatically when reaching the permissible increment |  |

# "Repair mode" function (RMS with ES) P.15:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - RMS deactivated - Device status display "Operation enabled" |  |
| 2 | Activate repair mode via RMS | - Device status display "Switch on disabled" - Safe torque off-display active |  |
| 3 | Reset error if necessary | No errors concerning safety engineering |  |
| 4 | Activate motion via ES and travel command | - Motor traverses - Safe torque off-display not active - No errors concerning safety engineering |  |
| 5 | Deactivate motion via ES | If SS1 is used as stop function - Motor decelerates along QSP application-ramp to standstill. - Device status display "Operation/quick stop active" After expiry of the monitoring time in ES for SS1 or if STO is used - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 7 | - Deactivate RMS - Acknowledge restart if necessary | - Safe torque off-display active - No errors concerning safety engineering |  |
| 8 | Traverse motor | Setpoint / actual value comparison |  |

# "Safe direction" function (SDI) P.16:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SDIpos and SDIneg deactivated - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor in positive direction | Setpoint / actual value comparison |  |
| 3 | Activate SDIneg function | - Motor decelerates along ramp to standstill - Device status display "Operation enabled" |  |
| 4 | Deactivate SDIneg | - Motor accelerates again - No errors concerning safety engineering |  |
| 5 | Traverse motor in negative direction | Setpoint / actual value comparison |  |
| 6 | Activate SDIpos function | - Motor decelerates along ramp to standstill - Device status display "Operation enabled" |  |
| 7 | Deactivate SDIpos | - Motor accelerates again - No errors concerning safety engineering |  |

# "Safe homing" function (SHom) P.17:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position not available - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor in the direction of the SHom_Start-mark | Setpoint / actual value comparison |  |
| 3 | SHom_Start-mark reached resp. SHom_Start via safety-bus | - SHom_active is set - Depending of the i950 homing mode the direction of motion can change into the direction of the SHom_Load-mark |  |
| 4 | SHom_Load-mark reached resp. SHom_Load via safety-bus | - SHom_active is reset - SHom_available is set |  |
| If the restart condition „load stored home position“ is used, after switching-on the mini-homing is necessary. |  |  |  |
| 5 | Initial status | - Home position not available - SHom_active is set - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 6 | Traverse motor | Setpoint / actual value comparison |  |
| 7 | fourfold travelling distance of „tolerance position comparison“ is reached | - SHom_active is reset - SHom_available is set |  |

If the same edge is used for SHom_Start and SHom_Load, step 3 will be skipped and when the SHom_Start-mark is reached directly SHom_available is set.

# "Safe cam 1" function (SCA) P.18:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint / actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safe cam 2" function (SCA) P.19:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint / actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safe cam 3" function (SCA) P.20:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint / actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safe cam 4" function (SCA) P.21:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Traverse motor in the direction of the cam-area | Setpoint / actual value comparison |  |
| 3 | Reaching the area | Safe output switches resp. bit on safety-bus switches on |  |
| 4 | Leaving the area | Safe output switches resp. bit on safety-bus switches off |  |

# "Safely-limited position 1" function (SLP) P.22:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Motor is within the position area, which must not be exceeded - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate SLP1 function | No errors concerning safety engineering |  |
| 3 | Traverse motor within the monitored area | Setpoint / actual value comparison |  |
| 4 | Deactivate SLP1 | No errors concerning safety engineering |  |
| 5 | Traverse motor out of the monitored area | - Setpoint / actual value comparison - No errors concerning safety engineering |  |

# "Safely-limited position 2" function (SLP) P.23:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Motor is within the position area, which must not be exceeded - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate SLP2 function | No errors concerning safety engineering |  |
| 3 | Traverse motor within the monitored area | Setpoint / actual value comparison |  |
| 4 | Deactivate SLP2 | No errors concerning safety engineering |  |
| 5 | Traverse motor out of the monitored area | - Setpoint / actual value comparison - No errors concerning safety engineering |  |

# "Safely-limited position 3" function (SLP) P.24:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Motor is within the position area, which must not be exceeded - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate SLP3 function | No errors concerning safety engineering |  |
| 3 | Traverse motor within the monitored area | Setpoint / actual value comparison |  |
| 4 | Deactivate SLP3 | No errors concerning safety engineering |  |
| 5 | Traverse motor out of the monitored area | - Setpoint / actual value comparison - No errors concerning safety engineering |  |

# "Safely-limited position 4" function (SLP) P.25:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Motor is within the position area, which must not be exceeded - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate SLP4 function | No errors concerning safety engineering |  |
| 3 | Traverse motor within the monitored area | Setpoint / actual value comparison |  |
| 4 | Deactivate SLP4 | No errors concerning safety engineering |  |
| 5 | Traverse motor out of the monitored area | - Setpoint / actual value comparison - No errors concerning safety engineering |  |

# "Position depending safe speed" function (PDSS) P.26:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Home position available - Motor is within the position area, which must not be exceeded - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate PDSS function (if not permanently active) | No errors concerning safety engineering |  |
| 3 | Traverse motor within the monitored area | Setpoint / actual value comparison |  |
| If the SCS-areas are used |  |  |  |
| 4 | Traverse motor into the negative SCS-area | Setpoint / actual value comparison |  |
| 5 | Traverse motor into the positive SCS-area | Setpoint / actual value comparison |  |

# “Safe brake control“ function (SBC) P.27:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate SBC function | - Brake is applied after the delay time has expired - Stop function is activated - No errors concerning safety engineering |  |
| 3 | Deactivate SBC function | - Stop function is deactivated - Brake releases after the delay time has expired - No errors concerning safety engineering |  |
| 4 | Traverse motor | Setpoint / actual value comparison |  |

# “Muting“ function P.28:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Muting password created - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate muting function | - Input muting password - No errors concerning safety engineering |  |
| 3 | Mute S-bus | All bits of the safety bus are deactivated |  |
| 4 | Mute SD-In1 | All functions of SD-In1 are deactivated |  |
| 5 | Mute SD-In2 | All functions of SD-In2 are deactivated |  |
| 6 | Mute SD-In3 | All functions of SD-In3 are deactivated |  |
| 7 | Mute SD-In4 | All functions of SD-In4 are deactivated |  |
| 8 | Deactivate muting | No errors concerning safety engineering |  |

# Error scenarios (validation through test according to EN ISO 13849-2) (negative testing)

The below-described error scenarios exclusively refer to the functionality of the ES-component. For further components contained in the safety chain, error scenarios must be checked as well.

At a series-commissioning with identical parameter sets the negative tests can be skipped from the 2nd axis on, because with these tests the correct input of the parameter values is checked. The sameness of the parameter sets can be verified by a comparison of the CRC’s.

# Safe homing via inputs IRS and IRL N.1

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Open circuit IRS: (remove connecting wire) - Move to the homing mark, so that SHom_Start would be set | SHom active is not set |  |
| 2 | Restore functionality of IRS | No errors concerning safety engineering |  |
| 3 | - Open circuit IRL: (remove connecting wire) - Set SHom_Start - Move to the homing mark, so that SHom_Load would be set | - SHom active is reset after the timeout - SHom available is not set - Error status display "SHom: Timeout" |  |
| 4 | - Restore functionality of IRL - Acknowledge error - Execute homing | No errors concerning safety engineering |  |

# Safe input 1 (SD-In1) N.2

Used: 		Not used:

Sensor type:

The following tests only have to be executed, if the input is parameterized as passive sensor.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In1, channel x“ |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - No errors concerning safety engineering |  |

# Safe input 2 (SD-In2) N.3

Used: 		Not used:

Sensor type:

The following tests only have to be executed, if the input is parameterized as passive sensor.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In2, channel x" |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - No errors concerning safety engineering |  |

# Safe input 3 (SD-In3) N.4

Used: 		Not used:

Sensor type:

The following tests only have to be executed, if the input is parameterized as passive sensor.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In3, channel x" |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - No errors concerning safety engineering |  |

# Safe input 4 (SD-In4) N.5

Used: 		Not used:

Sensor type:

The following tests only have to be executed, if the input is parameterized as passive sensor.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Short circuit between the disconnecting paths: (connect wire jumper) | - Parameterised safety function is activated latest after about 10s - Error status display "Stuck at High at SD-In4, channel x" |  |
| 2 | - Restore functionality of both disconnecting paths and activate sensor - Acknowledge error | - Error acknowledgement possible - No errors concerning safety engineering |  |

# "Safe stop 1" function (SSE, SS1 or special operation mode) N.6

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select deceleration ramp in basic device longer (decrease 0x500A:48 + 0x500A:49) than stopping time in ES - Activate SS1 | - Motor decelerates along the ramp - Device status display "Operation/quick stop active" Without ramp monitoring after expiry of the stopping time - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SS1/SS2: Stopping time exceeded" With ramp monitoring after exceeding the ramp - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SS1/SS2: Speed ramp exceeded" |  |
| 2 | - Reset deceleration ramp in basic device to the original values - Acknowledge error | No errors concerning safety engineering |  |

# "Safe stop 2" function (SS2 or special operation mode) N.7

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select deceleration ramp in basic device longer (decrease 0x500A:48 + 0x500A:49) than stopping time in ES - Activate SS2 | - Motor decelerates along the ramp - Device status display "Operation/quick stop active" Without ramp monitoring after expiry of the stopping time - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SOS: Tolerance limit exceeded" With ramp monitoring after exceeding the ramp - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SS1/SS2: Speed ramp exceeded" |  |
| 2 | - Reset error - Issue a motion command | - Motor traverses Exceeding the position tolerance - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SOS: Tolerance limit exceeded" |  |
| 3 | - Reset deceleration ramp in basic device to the original values - Acknowledge error | No errors concerning safety engineering |  |

# "Safe operating stop" function (SOS or special operation mode) N.8

(only necessary, if used independent from SS2)

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Activate SOS - Issue a motion command | - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SOS: Tolerance limit exceeded" |  |
| 2 | Acknowledge error | No errors concerning safety engineering |  |

# "Safely-limited speed 1" function (SLS1, SHom or special operation) N.9

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim1 in ES - Activate SLS1 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS1: Nlim1 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |
| 3 | - Select deceleration ramp in basic device longer than the braking time in ES - Activate SLS1 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS1: Nlim1 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safely-limited speed 2" function (SLS2, SHom or special operation) N.10

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim2 in ES - Activate SLS2 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS2: Nlim2 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |
| 3 | - Select deceleration ramp in basic device longer than the braking time in ES - Activate SLS2 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS2: Nlim2 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safely-limited speed 3" function (SLS3, SHom or special operation) N.11

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim3 in ES - Activate SLS3 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS3: Nlim3 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |
| 3 | - Select deceleration ramp in basic device longer than the braking time in ES - Activate SLS3 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS3: Nlim3 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safely-limited speed 4" function (SLS4, SHom or special operation) N.12

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select limit speed in basic device higher than Nlim4 in ES - Activate SLS4 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS4: Nlim4 exceeded" |  |
| 2 | - Reset limit speed in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |
| 3 | - Select deceleration ramp in basic device longer than the braking time in ES - Activate SLS4 | - Motor decelerates along the ramp After expiry of the braking time - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLS4: Nlim4 exceeded" |  |
| 4 | - Reset deceleration ramp in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safe maximum speed" function N.13

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Select setpoint speed in basic device higher than SMS in ES | - Motor accelerates to new setpoint - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SMS exceeded" |  |
| 2 | - Reset setpoint speed in basic device to original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safe direction" function N.14

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Set controller inhibit or activate STO | Controller is inhibited |  |
| 2 | - Activate SDIneg (positive direction is inhibited) - Move motor (manually) in positive direction (if necessary, release brake) | Error status display "SDIneg: Wrong direction" |  |
| 3 | Acknowledge error | No errors concerning safety engineering |  |
| 4 | - Activate SDIpos (negative direction is inhibited) - Move motor (manually) in negative direction (if necessary, release brake) | Error status display "SDIpos: Wrong direction" |  |
| 5 | Acknowledge error | No errors concerning safety engineering |  |

If the motor cannot be moved manually, traverse the motor applicatively in the inhibited direction. For this, it may be necessary to deactivate the limitation of direction (e.g. control of L_MC1P_Limiter).

# "Safely-limited increment" function in normal operation N.15

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Select the permissible number of increments in the basic device higher than SLI in ES | - Motor exceeds limit of permissible increments - Parameterised error response (STO, SS1, SS2) is activated - Error status display "SLI: Safely limited increment exceeded" |  |
| 2 | - Reset the permissible number of increments in the basic device to the original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safely-limited increment" function in special operation N.16

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Select the permissible number of increments in the basic device higher than SLI in ES | - Motor exceeds limit of permissible increments - Stop function of the operating mode selection is activated - Error status display "SLI: Safely limited increment exceeded" |  |
| 2 | - Reset the permissible number of increments in the basic device to the original value - Acknowledge error | No errors concerning safety engineering |  |

# "Safe homing" function N.17

Used: 		Not used:

| Step | Description |  | Expected status |  | Actual status |
| --- | --- | --- | --- | --- | --- |
| 1 | Initial status |  | - SHom is not available - SHom_Start- and / or SHom_Load-mark is manipulated, so that it can’t be detected |  |  |
| 2 | Traverse the motor to the mark, that SHom_available would be set |  | - Mark is not detected - SLSx stays activated - SHom_available is not set - Error status display "SHom: Timeout" |  |  |
| 3 | - Acknowledge error - Put the mark back into operation - Execute the homing procedure |  | No errors concerning safety engineering |  |  |
| If diagnostic marks are used |  |  |  |  |  |
| 4 | Initial status | - SHom is available - Diagnostic sensor is manipulated, so that the marks can’t be detected |  |  |  |
| 5 | Move the motor in the direction of the first diagnostic mark | - Mark is not detected - Error status display “SHom: Diagnostic mark detected in invalid position“ or “SHom: No diagnostic mark detected at expected position“ |  |  |  |
| 6 | - Acknowledge error - Move the motor in the direction of the second diagnostic mark | - Mark is not detected - Error status display ”SHom: Diagnostic mark detected in invalid position“ or “SHom: No diagnostic mark detected at expected position“ |  |  |  |
| 7 | - Acknowledge error - Put the mark back into operation |  | No errors concerning safety engineering |  |  |

# "Safely-limited position 1" function N.18

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SHom is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP1 function | No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP1: Position area exceeded“ |  |
| 4 | - Deactivate SLP1 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP1 function | No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP1: Position area exceeded“ |  |
| 7 | - Deactivate SLP1 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Safely-limited position 2" function N.19

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SHom is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP2 function | No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP2: Position area exceeded“ |  |
| 4 | - Deactivate SLP2 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP2 function | - No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP2: Position area exceeded“ |  |
| 7 | - Deactivate SLP2 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Safely-limited position 3" function N.20

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SHom is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP3 function | No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP3: Position area exceeded“ |  |
| 4 | - Deactivate SLP3 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP3 function | No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP3: Position area exceeded“ |  |
| 7 | - Deactivate SLP3 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Safely-limited position 4" function N.21

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SHom is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Activate the SLP4 function | No errors concerning safety engineering |  |
| 3 | Traverse the motor out of the position area in positive direction | Error status display „SLP4: Position area exceeded“ |  |
| 4 | - Deactivate SLP4 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |
| 5 | Activate the SLP4 function | No errors concerning safety engineering |  |
| 6 | Traverse the motor out of the position area in negative direction | Error status display „SLP4: Position area exceeded“ |  |
| 7 | - Deactivate SLP4 - Acknowledge error | - No errors concerning safety engineering - Traverse motor back into the position area |  |

# "Position-dependent safe speed" function N.22

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SHom is available - Motor is within the position area, which must not be exceeded |  |
| 2 | Traverse motor in positive direction | Error status display „PDSS: Speed limit exceeded“ when the monitoring curve is exceeded |  |
| 3 | Acknowledge error | No errors concerning safety engineering |  |
| 4 | Traverse motor in negative direction | Error status display „PDSS: Speed limit exceeded“ when the monitoring curve is exceeded |  |
| 5 | Acknowledge error | No errors concerning safety engineering |  |
| If the SCS-areas are used |  |  |  |
| 6 | Traverse motor in positive direction with speed above the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the SCS-area is reached |  |
| 7 | Acknowledge error | No errors concerning safety engineering |  |
| 8 | Traverse motor in positive direction with speed below the SCS-limit and then accelerate above the SCS-limit | Error status display „PDSS: Creeping speed SCS exceeded“ |  |
| 9 | Acknowledge error | No errors concerning safety engineering |  |
| 10 | Traverse motor in positive direction with speed below the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the traversing range is exceeded |  |
| 11 | - Deactivate PDSS or activate the repair mode - Acknowledge error | No errors concerning safety engineering |  |
| 12 | - Traverse motor back into the PDSS-area - Activate PDSS | No errors concerning safety engineering |  |
| 13 | Traverse motor in negative direction with speed above the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the SCS-area is reached |  |
| 14 | Acknowledge error | No errors concerning safety engineering |  |
| 15 | Traverse motor in negative direction with speed below the SCS-limit and then accelerate above the SCS-limit | Error status display „PDSS: Creeping speed SCS exceeded“ |  |
| 16 | Acknowledge error | No errors concerning safety engineering |  |
| 17 | Traverse motor in negative direction with speed below the SCS-limit | Error status display „PDSS: Speed limit exceeded“ when the traversing range is exceeded |  |
| 18 | - Deactivate PDSS or activate the repair mode - Acknowledge error | No errors concerning safety engineering |  |
| 19 | - Traverse motor back into the PDSS-area - Activate PDSS | No errors concerning safety engineering |  |

# "Safety-bus" N.23

Used: 		Not used:

If a safety bus and the operating mode / the repair mode is used, check if a stop function will be activated if a bus error occurs. When a bus error occurs, the ES interprets all activated safety-bus bits as FALSE. (FALSE activates the corresponding stop-/ monitoring function)

| Priority | Function |
| --- | --- |
| high | STO SS1 SS2 SLSx, SDI, SLI, SLP, PDSS, … |
| low |  |

The following stop function(s) is / are activated via the safety bus.

# Two-encoder concept N.24

Used: 		Not used:

If a two-encoder concept is used to detect the safe speed and safe position in ES, check whether the ES detects speed discrepancies exceeding the tolerances.

You have to activate phase control in the drive controller, if necessary. You have to manipulate the load encoder such that a speed and if applicable a position discrepancy can occur (e.g. by covering / interrupting the laser or dismounting the encoder). Then the drive can be operated without position evaluation of the load encoder. The encoder will still be evaluated by the ES.

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Traverse motor so that the speed-tolerance threshold is exceeded | - Motor traverses beyond tolerance limit - STO is activated - Error status display "SSM: Speed comparison error" |  |
| 2 | - Activate position control, if necessary - Restore evaluation of the two encoders - Acknowledge error | No errors concerning safety engineering |  |
| Only relevant, if functions with absolute position monitoring are used |  |  |  |
| 3 | Traverse motor so that the position-tolerance threshold is exceeded or provoke a jump of the load encoder position | - Motor traverses beyond tolerance limit - Parameterized stop function is activated - Error status display „Tolerance position comparison exceeded“ |  |
| 4 | - Activate position control, if necessary - Restore evaluation of the two encoders - Acknowledge error | No errors concerning safety engineering |  |

May not be tested, if the slip compensation is activated

# Protocol conclusion

Safety-oriented parameters			Check sum (CRC)

File name:      .bin					0x

Data back-up ES-component

|  | Storage medium |  |  | Storage location |
| --- | --- | --- | --- | --- |
|  | Type | Designation / version | Date |  |
| Parameters |  |  |  |  |
| Circuit diagrams |  |  |  |  |

Countersignatures

The 'commissioning engineer'

hereby confirms that the above-described tests and checks were carried out properly by skilled personnel.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |

The 'machine manufacturer'

hereby confirms that the printout of the parameterisation carried out (attachment of this protocol) will be compared for compliance with the manufacturer's own specification. The responsibility for the safety of the machine remains with the machine manufacturer.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |