Selecting the safety parameters for i950 with ES V1.5

Issued by:			.....

Machine / type:		.....

Systems / system part:	.....

Name of axis:			.....

| Password |  |  |
| --- | --- | --- |
| 0.1 Password (min. 6 characters) | Standard „Lenze Safety“ | Own ..... |
| 0.2 Muting password (min. 6 characters) | ..... |  |

| General |  |  |  |
| --- | --- | --- | --- |
| 1.1.1 Safety address |  | 1.1.2 Parameter set: Version | i950 ES V1.5 |

| Feedback system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.2.1 Motor encoder: System | No motor encoder |  |  |  | Sin-cos encoder |  |  |  |  |
|  | Resolver |  |  |  | HIPERFACE DSL |  |  |  |  |
| No entries required when "No motor encoder" is selected |  |  |  |  |  |  |  |  |  |
| 1.2.2 SinCos encoder PPR |  |  | 1.2.3 Number of resolver pole pairs |  |  |  |  |  |  |
| 1.2.4 Type key HIPERFACE DSL / OCT |  |  |  |  | EEx37-2xF0x017A / Ax17-8V-D2 |  |  |  |  |
| EKx35-2xF0B020A / Ax20-8V-D2 | EDx35-2xF0A020A / Ax20-8V-D2-1 |  |  |  | EDx35-2xF0A024A / Ax24-8V-D2 |  |  |  |  |
| 1.2.5 Motor mounting direction | Motor rotating CW |  |  |  | Motor rotating CCW |  |  |  |  |
| 1.2.6 Load encoder: System | No position encoder |  |  | Analog encoder |  |  | Digital encoder |  |  |
| No entries required when "No position encoder" is selected |  |  |  |  |  |  |  |  |  |
| 1.2.7 Load encoder: Gear numerator |  |  |  |  |  |  |  |  |  |
| 1.2.8 Load encoder: Gear denominator |  |  |  |  |  |  |  |  |  |
| 1.2.9 Load encoder: Additional gear numerator |  |  |  |  |  |  |  |  |  |
| 1.2.10 Load encoder: Additional gear denominator |  |  |  |  |  |  |  |  |  |
| 1.2.11 Load encoder: Mounting direction |  | As motor encoder |  |  |  | Inverted to motor encoder |  |  |  |
| 1.2.12 Load encoder: Error response |  |  |  |  |  | STO |  | SS1 |  |
| 1.2.13 Load encoder: End area error response |  |  |  |  |  | STO |  | SS1 |  |
| 1.2.14 Load encoder: Max. position jump |  |  |  |  |  |  |  | incr. |  |

| Speed aquisition |  |  |  |
| --- | --- | --- | --- |
| No entries required when "No motor encoder" is selected |  |  |  |
| 1.3.1 Tolerance window (n=0) |  | rpm |  |
| 1.3.2 Tolerance speed comparison |  | rpm |  |
| 1.3.3 Response time encoder monitoring | 12msec | 50msec | 100msec |
| 1.3.4 „n=0“ - Output | Deactivated | SD-Out1 positive logic |  |  |
|  |  | SD-Out1 negative logic |  |  |
| 1.3.5 „Positive direction“ - Output | Deactivated | SD-Out1 positive logic |  |  |
|  |  | SD-Out1 negative logic |  |  |

| Position aquisition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.4.1 Tolerance position comparison |  |  |  | incr. |  |
| 1.4.2 SHom_Start Source | Deactivated |  |  | S-bus | IRS |
| 1.4.3 SHom_Start: IRS edge trigger | Rising edge |  |  | Falling edge |  |
| 1.4.4 SHom_Load Source | S-bus |  |  | IRL |  |
| 1.4.5 SHom_Load: IRL edge trigger | Rising edge |  |  | Falling edge |  |
| 1.4.6 SHom: Home Position |  |  |  | incr. |  |
| 1.4.7 SHom: Timeout |  |  |  | msec |  |
| 1.4.8 SHom: SLS for homing | SLS1 | SLS2 |  | SLS3 | SLS4 |
| 1.4.9 SHom: Restart condition | Homing necessary |  |  |  |  |
|  | Load stored home position |  |  |  |  |
| 1.4.10 SHom: Starting position tolerance |  |  |  | incr. |  |
| 1.4.11 SHom Source diagnostisch positions | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 |  | SD-In3 |  |
|  | SD-In4 | IRS |  | IRL |  |
| 1.4.12 SHom: Level diagnostic positions | Low level |  |  | High level |  |
| 1.4.13 SHom: Lower diagnostic position |  |  |  | incr. |  |
| 1.4.14 SHom: Upper diagnostic position |  |  |  | incr. |  |
| 1.4.15 SHom: Tolerance diagnostic positions |  |  |  | incr. |  |
| 1.4.16 SHom: Error response diag. positions | STO | SS1 |  | SS2 |  |
| 1.4.17 SHom: Maximum compensated slip |  |  | incr. |  |  |
| 1.4.18 SHom: End area slip compensation | Deactivated |  | Activated |  |  |
| 1.4.19 „SHom active“ - Output | Deactivated |  | SD-Out1 positive logic |  |  |
|  |  |  | SD-Out1 negative logic |  |  |
| 1.4.20 „SHom available“ - Output | Deactivated |  | SD-Out1 positive logic |  |  |
|  |  |  | SD-Out1 negative logic |  |  |

| SD-In1 (Safe input 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1.1 Sensor type | Input deactivated |  | Passive sensor |  | Active sensor |  |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.1.2 Discrepancy time |  | msec | 2.1.3 Input delay |  |  |  | msec |
| 2.1.4 „SD-In1“ - Output |  |  | Deactivated | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |

| SD-In2 (Safe input 2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.2.1 Sensor type | Input deactivated |  | Passive sensor |  | Active sensor |  |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.2.2 Discrepancy time |  | msec | 2.2.3 Input delay |  |  |  | msec |
| 2.2.4 „SD-In2“ - Output |  |  | Deactivated | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |

| SD-In3 (Safe input 3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.3.1 Sensor type | Input deactivated |  | Passive sensor |  | Active sensor |  |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.3.2 Discrepancy time |  | msec | 2.3.3 Input delay |  |  |  | msec |
| 2.3.4 „SD-In3“ - Output |  |  | Deactivated | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |

| SD-In4 (Safe input 4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.4.1 Sensor type | Input deactivated |  | Passive sensor |  | Active sensor |  |  |
| No entries required when "Input deactivated" is selected |  |  |  |  |  |  |  |
| 2.4.2 Discrepancy time |  | msec | 2.4.3 Input delay |  |  |  | msec |
| 2.4.4 „SD-In4“ - Output |  |  | Deactivated | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |

| SD-Out1 (Sicherer Output 1) |  |  |  |
| --- | --- | --- | --- |
| 3.1 Source S-Bus | Deactivated | Activated |  |
| 3.2 Logic function | OR | AND |  |
| 3.3 „Error active“ - Output |  | Deactivated | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |

| Muting general |  |  |
| --- | --- | --- |
| 4.1 MUT: Mute S-bus | No muting | Mute completely |
| 4.2 MUT: Mute SD-In1 | No muting | Mute -> 1 |
| 4.3 MUT: Mute SD-In2 | No muting | Mute -> 1 |
| 4.4 MUT: Mute SD-In3 | No muting | Mute -> 1 |
| 4.5 MUT: Mute SD-In4 | No muting | Mute -> 1 |
| 4.6 „MUT active“ - Output | Deactivated | SD-Out1 positive logic |
|  |  | SD-Out1 negative logic |

| Safety bus general |  |  |
| --- | --- | --- |
| 5.1 S-Bus configuration | No safety bus | PROFIsafe / PROFINET 8 words |
|  | FSoE / EtherCAT fieldbus | FSoE / EtherCAT onboard |

| Safe torque off |  |  |  |  |
| --- | --- | --- | --- | --- |
| 6.1.1 STO Source SD-In | Deactivated |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |
| 6.1.2 STO Source S-bus | Deactivated |  | Activated |  |
| 6.1.3 STO: Restart | Acknowledged restart |  | Automatic restart |  |
| 6.1.4 „STO active“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |

| Safe stop 1/2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.1 SS1, SS2: Stopping time |  |  |  | msec |  |
| 6.2.2 SS1/SS2: Ramp monitoring | No ramp monitoring |  |  | Ramp monitoring active |  |
| 6.2.3 SS1, SS2: Ramp - Smoothing time |  |  |  | % |  |
| 6.2.4 SS1, SS2: Ramp - Offset mode | Relative |  |  | Absolute |  |
| 6.2.5 SS1, SS2: Ramp - Start-offset relative |  |  |  | % |  |
| 6.2.6 SS1, SS2: Ramp - Start-offset absolute |  |  |  | rpm |  |
| 6.2.7 SS1 Source SD-In | Deactivated |  |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |
| 6.2.8 SS1 Source S-bus | Deactivated |  | Activated |  |  |  |
| 6.2.9 SS1: Mode | STO after stop time |  |  | STO when n=0 |  |
| 6.2.10 SS1: Delay STO after n=0 |  |  |  | msec |  |
| 6.2.11 „SS1 active“ - Output | Deactivated |  |  | SD-Out1 positive logic |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |
| 6.2.12 SS2 Source SD-In | Deactivated |  |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |
| 6.2.13 SS2 Source S-Bus | Deactivated |  | Activated |  |  |  |
| 6.2.14 SS2: Mode | SOS after stop time |  |  | SOS when n=0 |  |
| 6.2.15 „SS2 active“ - Output | Deactivated |  |  | SD-Out1 positive logic |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |

| Safe operating stop |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1 SOS Source SD-In | Deactivated |  |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |
| 6.3.2 SOS Source S-bus | Deactivated |  | Activated |  |  |  |
| 6.3.3 SOS: Tolerance window (Delta p=0) |  |  |  | incr. |  |
| 6.3.4 SOS: Restart | Acknowledged restart |  |  | Automatic restart |  |
| 6.3.5 „SOS observed“ - Output | Deactivated |  |  | SD-Out1 positive logic |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |

| Safe stop emergency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 6.4.1 SSE Source SD-In | Deactivated |  |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |
| 6.4.2 SSE Source S-bus | Deactivated |  | Activated |  |  |  |
| 6.4.3 SSE: Emergency stop function | STO |  |  | SS1 |  |
| 6.4.4 „SSE active“ - Output | Deactivated |  |  | SD-Out1 positive logic |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |

| Safe speed (SMS, SLS1-4) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1.1 SMS: Maximum speed Nmax |  |  |  |  | rpm |  |  |  |
| 7.1.2 SMS: Reaction (n>Nmax) |  | STO | SS1 |  | SS2 |  |  |  |
| 7.1.3 „SMS observed“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |  |
| 7.1.4 SSM: Monitored speed |  |  |  | rpm |  |  |  |  |
| 7.1.5 „SSM within limits“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |  |
| 7.1.6 SLS1 - Source SD-In |  | Deactivated |  |  |  |  |  |  |  |
|  |  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |  |  |
| 7.1.7 SLS1 - Source S-bus |  | Deactivated |  | Activated |  |  |  |  |  |
| 7.1.8 SLS1 - Source internal |  | Deactivated |  | SDIpos active |  |  |  |  |  |  |
|  |  | SDIneg active |  | SLI active |  |  |  |  |  |  |
|  |  | OMS active |  | SSM within limits |  |  |  |  |  |  |
|  |  | SDIpos observed |  | SDIneg observed |  |  |  |  |  |  |
|  |  | SLP1 observed |  | SLP2 observed |  |  |  |  |  |  |
|  |  | SLP3 observed |  | SLP4 observed |  |  |  |  |  |  |
|  |  | SCA1 within limits |  | SCA2 within limits |  |  |  |  |  |  |
|  |  | SCA3 within limits |  | SCA4 within limits |  |  |  |  |  |  |
|  |  | PDSSpos observed |  | PDSSneg observed |  |  |  |  |  |  |
|  |  | Positive direction |  |  |  |  |  |  |  |  |
| 7.1.9 SLS1 - Invert source internal |  | no inversion |  | Invert |  |  |  |  |  |  |
| 7.1.10 SLS1 - Limited speed Nlim1 |  | rpm | 7.1.11 SLS1 - Braking time Nlim1 |  |  |  |  | msec |
| 7.1.12 SLS1 - Permitted direction | Both directions released |  | Positive direction released |  | Negative direction released |  |  |  |
| 7.1.13 SLS1 - Reaction (n>Nlim1) | STO |  | SS1 |  | SS2 |  |  |  |
| 7.1.14 „SLS1 active“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.15 „SLS1 observed“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.16 SLS2 - Source SD-In |  | Deactivated |  |  |  |  |  |  |  |
|  |  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |  |  |
| 7.1.17 SLS2 - Source S-bus |  | Deactivated |  | Activated |  |  |  |  |  |
| 7.1.18 SLS2 - Source internal |  | Deactivated |  | SDIpos active |  |  |  |  |  |  |
|  |  | SDIneg active |  | SLI active |  |  |  |  |  |  |
|  |  | OMS active |  | SSM within limits |  |  |  |  |  |  |
|  |  | SDIpos observed |  | SDIneg observed |  |  |  |  |  |  |
|  |  | SLP1 observed |  | SLP2 observed |  |  |  |  |  |  |
|  |  | SLP3 observed |  | SLP4 observed |  |  |  |  |  |  |
|  |  | SCA1 within limits |  | SCA2 within limits |  |  |  |  |  |  |
|  |  | SCA3 within limits |  | SCA4 within limits |  |  |  |  |  |  |
|  |  | PDSSpos observed |  | PDSSneg observed |  |  |  |  |  |  |
|  |  | Positive direction |  |  |  |  |  |  |  |  |
| 7.1.19 SLS2 - Invert source internal |  | no inversion |  | Invert |  |  |  |  |  |  |
| 7.1.20 SLS2 - Limited speed Nlim2 |  | rpm | 7.1.21 SLS2 - Braking time Nlim2 |  |  |  | msec |  |
| 7.1.22 SLS2 - Permitted direction | Both directions released |  | Positive direction released |  | Negative direction released |  |  |  |
| 7.1.23 SLS2 - Reaction (n>Nlim2) | STO |  | SS1 |  | SS2 |  |  |  |
| 7.1.24 „SLS2 active“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.25 „SLS2 observed“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.26 SLS3 - Source SD-In |  | Deactivated |  |  |  |  |  |  |
|  |  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |  |
| 7.1.27 SLS3 - Source S-bus |  | Deactivated |  | Activated |  |  |  |  |
| 7.1.28 SLS3 - Source internal |  | Deactivated |  | SDIpos active |  |  |  |  |  |  |
|  |  | SDIneg active |  | SLI active |  |  |  |  |  |  |
|  |  | OMS active |  | SSM within limits |  |  |  |  |  |  |
|  |  | SDIpos observed |  | SDIneg observed |  |  |  |  |  |  |
|  |  | SLP1 observed |  | SLP2 observed |  |  |  |  |  |  |
|  |  | SLP3 observed |  | SLP4 observed |  |  |  |  |  |  |
|  |  | SCA1 within limits |  | SCA2 within limits |  |  |  |  |  |  |
|  |  | SCA3 within limits |  | SCA4 within limits |  |  |  |  |  |  |
|  |  | PDSSpos observed |  | PDSSneg observed |  |  |  |  |  |  |
|  |  | Positive direction |  |  |  |  |  |  |  |  |
| 7.1.29 SLS3 - Invert source internal |  | no inversion |  | Invert |  |  |  |  |  |  |
| 7.1.30 SLS3 - Limited speed Nlim3 |  | rpm | 7.1.31 SLS3 - Braking time Nlim3 |  |  |  | msec |  |
| 7.1.32 SLS3 - Permitted direction | Both directions released |  | Positive direction released |  | Negative direction released |  |  |  |
| 7.1.33 SLS3 - Reaction (n>Nlim3) | STO |  | SS1 |  | SS2 |  |  |  |
| 7.1.34 „SLS3 active“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.35 „SLS3 observed“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.36 SLS4 - Source SD-In |  | Deactivated |  |  |  |  |  |  |
|  |  | SD-In1 | SD-In2 | SD-In3 |  | SD-In4 |  |  |
| 7.1.37 SLS4 - Source S-bus |  | Deactivated |  | Activated |  |  |  |  |
| 7.1.38 SLS4 - Source internal |  | Deactivated |  | SDIpos active |  |  |  |  |  |  |
|  |  | SDIneg active |  | SLI active |  |  |  |  |  |  |
|  |  | OMS active |  | SSM within limits |  |  |  |  |  |  |
|  |  | SDIpos observed |  | SDIneg observed |  |  |  |  |  |  |
|  |  | SLP1 observed |  | SLP2 observed |  |  |  |  |  |  |
|  |  | SLP3 observed |  | SLP4 observed |  |  |  |  |  |  |
|  |  | SCA1 within limits |  | SCA2 within limits |  |  |  |  |  |  |
|  |  | SCA3 within limits |  | SCA4 within limits |  |  |  |  |  |  |
|  |  | PDSSpos observed |  | PDSSneg observed |  |  |  |  |  |  |
|  |  | Positive direction |  |  |  |  |  |  |  |  |
| 7.1.39 SLS4 - Invert source internal |  | no inversion |  | Invert |  |  |  |  |  |  |
| 7.1.40 SLS4 - Limited speed Nlim4 |  | rpm | 7.1.41 SLS4 - Braking time Nlim4 |  |  |  | msec |  |
| 7.1.42 SLS4 - Permitted direction | Both directions released |  | Positive direction released |  | Negative direction released |  |  |  |
| 7.1.43 SLS4 - Reaction (n>Nlim4) | STO |  | SS1 |  | SS2 |  |  |  |
| 7.1.44 „SLS4 active“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |
| 7.1.45 „SLS4 observed“ - Output |  | Deactivated |  | SD-Out1 positive logic |  |  |  |  |
|  |  |  |  | SD-Out1 negative logic |  |  |  |  |

| Safe direction |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.2.1 SDIpos - Source SD-In |  | Deactivated |  |  |  |  |  |  |
|  |  | SD-In1 |  | SD-In2 |  | SD-In3 | SD-In4 |  |
| 7.2.2 SDIneg - Source SD-In |  | Deactivated |  |  |  |  |  |  |
|  |  | SD-In1 |  | SD-In2 |  | SD-In3 | SD-In4 |  |
| 7.2.3 SDIpos - Source S-bus |  | Deactivated |  |  |  | Activated |  |  |
| 7.2.4 SDIneg - Source S-bus |  | Deactivated |  |  |  | Activated |  |  |
| 7.2.5 SDI: Monitoring standard operation | Both directions released |  |  |  |  |  |  |
|  | Positive direction released |  |  |  | Negative direction released |  |  |
| 7.2.6 SDI: Delay time |  |  |  |  |  | msec |  |
| 7.2.7 SDI: Tolerance threshold |  |  |  |  |  | incr. |  |
| 7.2.8 SDI: Error reaction |  | STO | SS1 |  |  | SS2 |  |
| 7.2.9 „SDIpos observed“ - Output |  | Deactivated |  |  |  | SD-Out1 positive logic |  |  |
|  |  |  |  |  |  | SD-Out1 negative logic |  |  |
| 7.2.10 „SDIneg observed“ - Output |  | Deactivated |  |  |  | SD-Out1 positive logic |  |  |
|  |  |  |  |  |  | SD-Out1 negative logic |  |  |

| Safely limited increment |  |  |  |  |
| --- | --- | --- | --- | --- |
| 7.3.1 SLI Source SD-In | Deactivated |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |
| 7.3.2 SLI Source S-bus | Deactivated |  | Activated |  |
| 7.3.3 SLI: Increment size standard op. |  |  | incr. |  |
| 7.3.4 SLI: Error response standard op. | STO | SS1 | SS2 |  |
| 7.3.5 SLI: Increment size special op. |  |  | incr. |  |
| 7.3.6 “SLI active” - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |

| Safely limited position |  |  |  |  |
| --- | --- | --- | --- | --- |
| 7.4.1 SLP1 - Source SD-In | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |  |
| 7.4.2 SLP1 - Source S-bus | Deactivated |  | Activated |  |  |
| 7.4.3 SLP1 - Lower position limit |  |  | incr. |  |
| 7.4.4 SLP1 - Upper position limit |  |  | incr. |  |
| 7.4.5 SLP1 - Error response | STO | SS1 | SS2 |  |
| 7.4.6 „SLP1 observed“ - Output | Deactivated |  | SD-Out1 positive logic |  |  |
|  |  |  | SD-Out1 negative logic |  |  |
| 7.4.7 SLP2 - Source SD-In | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |  |
| 7.4.8 SLP2 - Source S-bus | Deactivated |  | Activated |  |  |
| 7.4.9 SLP2 - Lower position limit |  |  | incr. |  |
| 7.4.10 SLP2 - Upper position limit |  |  | incr. |  |
| 7.4.11 SLP2 - Error response | STO | SS1 | SS2 |  |
| 7.4.12 „SLP2 observed“ - Output | Deactivated |  | SD-Out1 positive logic |  |  |
|  |  |  | SD-Out1 negative logic |  |  |
| 7.4.13 SLP3 - Source SD-In | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |  |
| 7.4.14 SLP3 - Source S-bus | Deactivated |  | Activated |  |  |
| 7.4.15 SLP3 - Lower position limit |  |  | incr. |  |
| 7.4.16 SLP3 - Upper position limit |  |  | incr. |  |
| 7.4.17 SLP3 - Error response | STO | SS1 | SS2 |  |
| 7.4.18 „SLP3 observed“ - Output | Deactivated |  | SD-Out1 positive logic |  |  |
|  |  |  | SD-Out1 negative logic |  |  |
| 7.4.19 SLP4 - Source SD-In | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |  |
| 7.4.20 SLP4 - Source S-bus | Deactivated |  | Activated |  |  |
| 7.4.21 SLP4 - Lower position limit |  |  | incr. |  |
| 7.4.22 SLP4 - Upper position limit |  |  | incr. |  |
| 7.4.23 SLP4 - Error response | STO | SS1 | SS2 |  |
| 7.4.24 „SLP4 observed“ - Output | Deactivated |  | SD-Out1 positive logic |  |  |
|  |  |  | SD-Out1 negative logic |  |  |

| Safe cam |  |  |
| --- | --- | --- |
| 7.5.1 SCA1 - Lower position limit |  | incr. |
| 7.5.2 SCA1 - Upper position limit |  | incr. |
| 7.5.3 „SCA1 within limits“ - Output | Deactivated | SD-Out1 positive logic |  |
|  |  | SD-Out1 negative logic |  |
| 7.5.4 SCA2 - Lower position limit |  | incr. |
| 7.5.5 SCA2 - Upper position limit |  | incr. |
| 7.5.6 „SCA2 within limits“ - Output | Deactivated | SD-Out1 positive logic |  |
|  |  | SD-Out1 negative logic |  |
| 7.5.7 SCA3 - Lower position limit |  | incr. |
| 7.5.8 SCA3 - Upper position limit |  | incr. |
| 7.5.9 „SCA3 within limits“ - Output | Deactivated | SD-Out1 positive logic |  |
|  |  | SD-Out1 negative logic |  |
| 7.5.10 SCA4 - Lower position limit |  | incr. |
| 7.5.11 SCA4 - Upper position limit |  | incr. |
| 7.5.12 „SCA4 within limits“ - Output | Deactivated | SD-Out1 positive logic |  |
|  |  | SD-Out1 negative logic |  |

| Position dependent safe speed (PDSS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 7.6.1 PDSS Source SD-In | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |  |
| 7.6.2 PDSS Source S-Bus | Deactivated |  | Activated |  |  |
| 7.6.3 PDSS: Permanent activation | On demand |  | Activated |  |
| 7.6.4 PDSS: Lower position limit |  |  | incr. |  |
| 7.6.5 PDSS: Upper position limit |  |  | incr. |  |
| 7.6.6 PDSS: Lower SCS limit |  |  | incr. |  |
| 7.6.7 PDSS: Upper SCS limit |  |  | incr. |  |
| 7.6.8 PDSS: SCS from lower limit |  |  | rpm |  |
| 7.6.9 PDSS: SCS from upper limit |  |  | rpm |  |
| 7.6.10 PDSS: Maximum speed |  |  | rpm |  |
| 7.6.11 PDSS: Maximum deceleration lower limit |  |  | rpm/s |  |
| 7.6.12 PDSS: Maximum deceleration upper limit |  |  | rpm/s |  |
| 7.6.13 PDSS: Error response | STO | SS1 | SS2 |  |
| 7.6.14 „PDSSpos observed“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |
| 7.6.15 „PDSSneg observed“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |

| Safe brake control (SBC) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 7.7.1 SBC Source SD-In | Deactivated |  |  |  |  |
|  | SD-In1 | SD-In2 | SD-In3 | SD-In4 |  |
| 7.7.2 SBC Source S-Bus | Deactivated |  | Activated |  |  |
| 7.7.3 SBC: Brake mode | SBC without STO |  | Automatic with STO |  |
| 7.7.4 SBC: Delay SBC |  |  | msec |  |
| 7.7.5 SBC: Brake open delay |  |  | msec |  |
| 7.7.6 SBC: Delay STO |  |  | msec |  |
| 7.7.7 „SBC active“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |
| 7.7.8 „SBC activated“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |

| Cascading function |  |  |
| --- | --- | --- |
| 7.8 CAS Source SD-In | Deactivated | SD-In4 |

| Operating mode selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| 7.9.1 OMS Source | Deactivated |  | SD-In1 | SD-In2 |
|  | S-Bus |  | SD-In3 | SD-In4 |
| 7.9.2 OMS: Stop function | STO | SS1 | SS2 |  |
| 7.9.3 OMS: Motion function | Free traversing |  |  |  |
|  | SLS1 | SLS2 | SLS3 | SLS4 |
| 7.9.4 OMS: Function at low level | Std. operation |  | Spcl. operation |  |
| 7.9.5 “OMS active” - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |
| 7.9.6 RMS Source | Deactivated |  | SD-In1 | SD-In2 |
|  | S-Bus |  | SD-In3 | SD-In4 |
| 7.9.7 RMS: Stop function | STO |  | SS1 |  |
| 7.9.8 „RMS active“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |
| 7.9.9 ES Source | Deactivated |  | SD-In1 | SD-In2 |
|  | S-Bus |  | SD-In3 | SD-In4 |
| 7.9.10 „ES active“ - Output | Deactivated |  | SD-Out1 positive logic |  |
|  |  |  | SD-Out1 negative logic |  |