Acceptance protocol for Advanced Safety

Fill in the acceptance protocol separately for every single drive controller with Advanced Safety V1.4.

# Overview

| Acceptance test no. |  |
| --- | --- |
| Date | 01.01.2025 |
| Carried out by: |  |

# Machine description and general picture

| Manufacturer |  |
| --- | --- |
| Machine / type / number |  |
| System / system part |  |
| Name of axis |  |
| Drive controller type (0x2000:001) |  |
| Serial number of drive controller (0x2000:002) |  |
| Version parameter set (0x2114:001) | i750 AS V1.4 |
| Serial number of safety component (0x212D:003) |  |
| FSoE-address |  |
| Circuit diagram version / relevant drawing sheets |  |
| Final customer |  |
| General picture of the machine |  |
| General picture of the safety circuit |  |

# Function check (positive testing)

Description

Carry out the function check separately for every single motor (as far as permitted by the machine).

If possible, application error messages should be switched to "No response" for the check to ensure that only error messages of the safety component will be displayed.

Execution

| First commissioning | Please tick |  |
| --- | --- | --- |
| Recommissioning (e.g. after disassembly for machine transport) |  |  |
| Serial commissioning |  |  |
| Function change |  |  |

Use the parameters displayed in 0x2114-28E9 and 0xF980 to check the selected parameters. Print out the selected parameters using the print function of the PLC Designer or EASY Starter and attach the printout to the acceptance protocol.

Mark functions that are not used as "Not used". Then the corresponding table can be deleted.

# "Safe torque off" function (STO) P.1:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - STO deactivated - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate STO function | - Drive coasts down or is decelerated by a mechanical brake. - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 4 | Deactivate STO | - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 5 | Traverse motor | Setpoint / actual value comparison |  |

# "Safe stop 1" function (SS1) P.2:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - SS1 deactivated - Device status display "Operation enabled" - Safe torque off-display not active - No errors concerning safety engineering |  |
| 2 | Traverse motor | Setpoint / actual value comparison |  |
| 3 | Activate SS1 function | - Motor decelerates along a ramp to standstill. - Device status display "Operation/quick stop active" if applicable After expiry of the monitoring time in AS - Drive switches to STO - Device status display "Switch on disabled" - Safe torque off-display active - No errors concerning safety engineering |  |
| 4 | Deactivate SS1 | - Device status display "Operation enabled” - Safe torque off-display not active - No errors concerning safety engineering |  |
| 5 | Traverse motor | Setpoint / actual value comparison |  |

# "Restart inhibit" function (STO) P.3:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | STO activated |  |
| 2 | Reset activation signal | Setpoint / actual value comparison |  |
| 3 | Acknowledge restart inhibit | STO deactivated |  |

# “Safe brake control“ function (SBC) P.4:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate SBC function | - Brake is applied after the delay time has expired - Stop function is activated - No errors concerning safety engineering |  |
| 3 | Deactivate SBC function | - Stop function is deactivated - Brake releases after the delay time has expired - No errors concerning safety engineering |  |
| 4 | Traverse motor | Setpoint / actual value comparison |  |

# “Muting“ function P.5:

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | Initial status | - Muting password created - Device status display "Operation enabled" - No errors concerning safety engineering |  |
| 2 | Activate muting function | - Input muting password - No errors concerning safety engineering |  |
| 3 | Mute S-bus | All bits of the safety bus are deactivated |  |
| 4 | Deactivate muting | No errors concerning safety engineering |  |

# Error scenarios (validation through test according to EN ISO 13849-2) (negative testing)

The below-described error scenarios exclusively refer to the functionality of the AS-component. For further components contained in the safety chain, error scenarios must be checked as well.

At a series-commissioning with identical parameter sets the negative tests can be skipped from the 2nd axis on, because with these tests the correct input of the parameter values is checked. The sameness of the parameter sets can be verified by a comparison of the CRC’s.

# "Safe stop 1" function (SS1) N.1

Used: 		Not used:

| Step | Description | Expected status | Actual status |
| --- | --- | --- | --- |
| 1 | - Select deceleration ramp longer than stopping time in AS - Activate SS1 | - Motor decelerates along the ramp - Device status display "Operation/quick stop active" if applicable - Drive coasts down or is decelerated and held by a mechanical brake - Device status display "Switch on disabled" - Error status display "SS1/SS2: Stopping time exceeded" |  |
| 2 | - Reset deceleration ramp to the original values - Acknowledge error | No errors concerning safety engineering |  |

# "Safety-bus" N.2

Used: 		Not used:

It must be checked if a stop function will be activated if a bus error occurs. When a bus error occurs, the AS component interprets all activated safety-bus bits as FALSE. (FALSE activates the corresponding stop-/ monitoring function)

| Priority | Function |
| --- | --- |
| high | STO SS1 |
| low |  |

The following stop function(s) is / are activated via the safety bus.

# Protocol conclusion

Safety-oriented parameters			Check sum (CRC)

File name:      .bin					0x

Data back-up AS-component

|  | Storage medium |  |  | Storage location |
| --- | --- | --- | --- | --- |
|  | Type | Designation / version | Date |  |
| Parameters |  |  |  |  |
| Circuit diagrams |  |  |  |  |

Countersignatures

The 'commissioning engineer'

hereby confirms that the above-described tests and checks were carried out properly by skilled personnel.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |

The 'machine manufacturer'

hereby confirms that the printout of the parameterisation carried out (attachment of this protocol) will be compared for compliance with the manufacturer's own specification. The responsibility for the safety of the machine remains with the machine manufacturer.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |