Functional enhancements and corrected functional limitations 
EASY Starter V1.27

Contents

1.	Functional extensions	3

1.1.	Product  EASY Starter 1.27	3

1.1.1.	c4xx, c5xx: SysLog (System Logging Protocol) is supported.	3

1.1.2.	c4xx, c5xx: OPC UA support extended.	3

1.1.3.	i950: Safety validation improved.	4

1.1.4.	i950: Settings dialogue for the security firewall.	4

1.1.5.	i950: TA Winder Dancer setting dialogues extended.	5

1.1.6.	i950: Setting dialogues for motor and application feedback improved.	5

1.1.7.	i750, i950: Kinematics setting dialogues improved.	6

1.1.8.	i750, i950: Setting dialogues for EnDat 2.1 SinCos and EnDat 2.2	6

1.1.9.	i950: Extension of the motor selection assistant	6

1.1.10.	i550, i650 motec application IO: IO-Link devices can be parameterised.	7

1.1.11.	i550, i650 application IO: Extended use of terminal X3.3 is supported.	7

1.1.12.	i550, i650 EtherNet IP CIP Safety is supported.	8

1.1.13.	i5xx, i6xx: Oscilloscope extension	8

1.2.	Product EASY Application Loader 1.27	9

1.2.1.	Optional reboot after downloading the application.	9

2.	Corrected functional restrictions .	10

2.1.	Product name - EASY Starter	10

2.1.1.	Exception error when connecting to i750 Double Inverter via "Ethernet - EtherCAT via Lenze Controller x3xx/c32xx".	10

2.1.2.	Incorrect indication that no suitable device description is available.	10

2.1.3.	Safety parameter CSV export deviating or incomplete	11

2.1.4.	Timestamp in logbook export incorrect	11

2.1.5.	"IoT configuration" tab visible although no longer required.	11

2.1.6.	Exception error in the "Select encoder" dialogue.	12

2.1.7.	Progress bar in the oscilloscope no longer shows the progress.	12

2.1.8.	EASY Starter only adds the first online device to the device list.	12

2.1.9.	Inconsistent IODD repository after deleting IODD.	13

2.1.10.	Exception error for connection "Ethernet - EtherCAT via Lenze Controller x3xx/c32xx.	13

2.1.11.	SM302-Safety parameter list: Error in plausibility check	14

2.1.12.	CSV format of the safe parameters differs from the format of the grey parameters	14

2.1.13.	Exception error when connecting via "Ethernet - EtherCAT to Lenze Controller c3xx/c32xx"	15

2.1.14.	Error in plausibility check SM302 for OMS with "low level"	15

2.1.15.	Communication driver continues to run even though the EASY Starter has been stopped	16

2.1.16.	If several LDA files are opened at the same time, the Easy Starter only loads the first online device	16

2.1.17.	Installation may fail	17

2.1.18.	Add 8400 motec only not possible over ADS EtherCAT	17

2.2.	Product name - EASY Firmware Loader	18

2.2.1.	8400-Topline: Firmware cannot be loaded in boot mode	18

2.2.2.	Exception after confirmation of an safety validation test	18

2.2.3.	Unhandled exception in case of invalid data in "Select Encoder" wizard	19

3.	Compatibility with previous versions	20

# Functional extensions

The functional enhancements listed here refer to the delivery versions described in the following table.

## Product	 EASY Starter 1.27

| order Designation (type) | Number of the drawing (internal) | Brief description |
| --- | --- | --- |
| On the Internet for download | Version 1.27 | EASY Starter 1.27 |

### c4xx, c5xx: SysLog (System Logging Protocol) is supported.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the SysLog (System Logging Protocol) communication function can now be parameterized if the firmware version of the controller supports this.

SysLog is a standard for logging and transmitting log messages in an IT and 0T network. The message severity levels to be sent can be set.

(SysE-23133)

Applications

Parameterization of a controller c4xx, c5xx.

Type of publication:

Public (AKB on the Internet/Intranet)

### c4xx, c5xx: OPC UA support extended.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the OPC UA PubSub and OPC UA Client communication function can now be parameterized if the firmware version of the controller supports this.

The parameterization of the OPC UA server in the controller has been expanded to include the GlobalDiscoveryServer (GDS) function.

A Global Discover Server (GDS) is an OPC UA server that enables clients to search for servers within the management domain.

(SysE-23133)

Applications

Parameterization of the OPC UA function of a controller c4xx, c5xx.

Type of publication:

Public (AKB on the Internet/Intranet)

### i950: Safety validation improved.

Which products are affected?

EASY Starter 1.27

Functional description

Safety validation has been improved with EASY Starter V1.27.

Usability has been improved with many small changes. Further usability improvements are in preparation for the next version.

(SysE-20801)

Applications

Validation of the extended safety functions of the i950.

Type of publication:

Public (AKB on the Internet/Intranet)

### i950: Settings dialogue for the security firewall.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the firewall, which is available from V1.11 of the i950, can be parameterized via a settings dialogue.

(SysE-22746)

Applications

Use of the i950 firewall to restrict access to the i950 via various interfaces.

Type of publication:

Public (AKB on the Internet/Intranet)

### i950: TA Winder Dancer setting dialogues extended.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the setting dialogues of the TA Winder Dancer have been extended for the new "VelocityMaster" dancer control method.

The analogue extension of the TA Winder Tension will be available in a subsequent version.

(SysE-21224)

Applications

Utilization of the "VelocityMaster" dancer control method in the i950 TA Winder Dancer inverter.

Type of publication:

Public (AKB on the Internet/Intranet)

### i950: Setting dialogues for motor and application feedback improved.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the setting dialogues for motor and application feedback have been expanded. In particular, the parameterization of linear motor encoders has been improved.

(SysE-21262 & SysE-21251)

Applications

Parameterization of motor and application feedback of the i950 inverter.

Type of publication:

Public (AKB on the Internet/Intranet)

### i750, i950: Kinematics setting dialogues improved.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the setting dialogues for the kinematics have been improved.

With newer versions of the i750 and i950 TA CiA402 inverters, the kinematics can also be parameterized.

Knowledge of the kinematics is required for a user-friendly setting of the application feedback encoders for which default values are now calculated. 
The dialogues of i750 and i950 have been aligned as closely as possible.

(SysE-21261 & SysE-20926 & 20924)

Applications

Parameterization of the kinematics of the i750 & i950 inverters.

Type of publication:

Public (AKB on the Internet/Intranet)

### i750, i950: Setting dialogues for EnDat 2.1 SinCos and EnDat 2.2

EASY Starter 1.27

Functional description

The EASY Starter V1.27 has been expanded to include special setting dialogues for EnDat 2.1 SinCos and EnDat 2.2 encoders.

(SysE-22475)

Applications

Use of encoder types EnDat 2.1 SinCos or EnDat 2.2 on inverter i750 & i950.

Type of publication:

Public (AKB on the Internet/Intranet)

### i950: Extension of the motor selection assistant

EASY Starter 1.27

Functional description

Newer versions of i750 and i950 support two saturation characteristics.

The second characteristic curve is often not included in the current motor description.

For newer motors, this characteristic is now part of the motor description and can be written to the inverter when a new motor is selected in the motor wizard. EASY Starter 1.27 now also parameterized the 2nd saturation characteristic if the inverter supports this and the motor description contains this characteristic.

(SysE-22875)

Applications

Use of newer Lenze motors on the i750 & i950 inverter.

Type of publication:

Public (AKB on the Internet/Intranet)

### i550, i650 motec application IO: IO-Link devices can be parameterised.

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the IO-Link devices connected to the i550 or 650 motec application IO can now also be parameterized. A parameter list with all parameters of the associated IODD (IO-Link Device Description) is available for this purpose.
(SysE-23175)

Applications

Use of an i550 or i650 motec with connected IO-Link devices.

Type of publication:

Public (AKB on the Internet/Intranet)

### i550, i650 application IO: Extended use of terminal X3.3 is supported.

Which products are affected?

EASY Starter 1.27

Functional description

EASY Starter V1.27 now supports the functional extension from V4.4 on terminal X3.3. All functions of this terminal can be selected in the setting dialogues.

(SysE-22894)

Applications

Use of an i550 or i650 motec application IO >= V4.4.

Type of publication:

Public (AKB on the Internet/Intranet)

### i550, i650 EtherNet IP CIP Safety is supported.

Which products are affected?

EASY Starter 1.27

Functional description

EASY Starter V1.27 now also supports the i550 and i650 motec variants with CIP Safety. There is an additional tab card for safe parameterization.

Additional diagnostics for safe parameterization are also available.

(SysE-23173)

Applications

Use of an i550 or i650 motec EtherNet IP CIP Safety.

Type of publication:

Public (AKB on the Internet/Intranet)

### i5xx, i6xx: Oscilloscope extension

Which products are affected?

EASY Starter 1.27

Functional description

With EASY Starter V1.27, the oscilloscope can now also be triggered with edges of Boolean signals.

(SysE-23169)

Applications

Using the oscilloscope of an i5xx or i6xx.

Type of publication:

Public (AKB on the Internet/Intranet)

## Product EASY Application Loader 1.27

| order Designation (type) | Number of the drawing (internal) | Brief description |
| --- | --- | --- |
| Available for download on the Internet as part of the EASY Starter Setup | Version 1.27 | EASY Application Loader 1.27 |

### Optional reboot after downloading the application.

Which products are affected?

EASY Application Loader 1.27

Functional description

After downloading the application, the device can optionally be restarted.

(SysE-21274)

Applications

Restart after application download.

Type of publication:

Public (AKB on the Internet/Intranet)

# Corrected functional restrictions .

The corrected functional restrictions listed here refer to the delivery versions listed in the table below.

## Product name - EASY Starter

| order Designation (type) | Number of the drawing (internal) | Brief description |
| --- | --- | --- |
| On the Internet for download | Version 1.27 | EASY Starter 1.27 |

### Exception error when connecting to i750 Double Inverter via "Ethernet - EtherCAT via Lenze Controller x3xx/c32xx".

Problem:

If the user attempts to add an i750 Double Inverter to the device list online via an "Ethernet - EtherCAT via Lenze Controller x3xx/c32xx" connection, the action fails with one exception.

(2200003714, L-5589)

Remedy:

The problem has been fixed.

Type of publication:

Public (AKB on the Internet/Intranet)

### Incorrect indication that no suitable device description is available.

Problem:

If no PLC application is available in a controller, a message is displayed in EASY Starter after online addition to the device list that no suitable device description is available.

(L-5331)

Remedy:

The problem has been fixed. The EASY Package Manager device description is used, and a warning is no longer displayed.

Type of publication:

Public (AKB on the Internet/Intranet)

### Safety parameter CSV export deviating or incomplete

Problem:

1. the columns of the exported csv file for the safety parameters are titled differently to the columns of the exported csv file for the grey parameters.

2. the columns "Raw value" and "Raw value hex" do not contain any values.

(2200003409, L-4567)

Remedy:
The "Export safe parameter set" function on the "Safety parameter list" tab has been changed:

Problem 1 has been fixed.

Problem 2 cannot be solved for architectural reasons.

(Background: This data is not available in the component released by TÜV and therefore cannot be exported).

Type of publication:

Public (AKB on the Internet/Intranet)

### Timestamp in logbook export incorrect

Problem:

The timestamp in the exported file does not contain the seconds.

(L-5914)

Remedy:

The problem has been fixed.

Type of publication:

Public (AKB on the Internet/Intranet)

### "IoT configuration" tab visible although no longer required.

Problem:

The "IoT configuration" tab is visible, although the required Lenze OPC UA server can no longer be installed.

(neither with the current version of EASY Starter nor separately).

(L-5507)

Remedy:

The "IoT configuration" tab has been removed and is no longer displayed.

Type of publication:

Public (AKB on the Internet/Intranet)

### Exception error in the "Select encoder" dialogue.

Problem:

When closing and then opening the "Select encoder" dialogue, an exception error may occur if the dialogue was closed with invalid entries in the filter.

(L-5507)

Remedy:

The error has been fixed.

Type of publication:

Public (AKB on the Internet/Intranet)

### Progress bar in the oscilloscope no longer shows the progress.

Problem:

The progress bar in the status bar of the oscilloscope stops without taking the progress of the recording into account. This makes it look as if the EASY Starter is stuck during longer recordings.

(L-6093)

Remedy:

The error has been fixed.

Type of publication:

Public (AKB on the Internet/Intranet)

### EASY Starter only adds the first online device to the device list.

Problem:

If several LDA files are opened at the same time, the EASY Starter only loads the first online device.

(L-5657)

Remedy:

The error has been fixed.

Type of publication:

Public (AKB on the Internet/Intranet)

### Inconsistent IODD repository after deleting IODD.

Problem:

If the user deletes an IO-Link Device Description (IODD) in EASY Package Manager while this IODD is still used in EASY Starter, an inconsistent status can be achieved in the IODD repository.

Additional information:
The user is informed that the action may lead to unexpected behavior, but he can do it anyway.

Consequences:

1. the user can no longer completely delete this IODD in the EASY Package Manager (even if he has closed the EASY Starter in the meantime)

2. reinstalling the IODD to resolve the problem is also not possible.

3. an inverter with an IO-Link device cannot be added online to the EASY Starter.

4. if the affected IO-Link device is selected in the EASY Starter wizard, an exception error message appears.

(L-6213)

Remedy:

The error has been fixed.

The user can repair the damaged IODD via the EASY Package Manager by reinstalling it.

An inverter with a connected IO-Link device can be inserted into the EASY Starter online despite a damaged IODD.

The wizard in the EASY Starter no longer generates an exception error message if the IODD is damaged.

Type of publication:

Public (AKB on the Internet/Intranet)

### Exception error for connection "Ethernet - EtherCAT via Lenze Controller x3xx/c32xx.

Problem:

The Easy Starter causes an exception error when an i750 double axis is added to the device list online via "Ethernet - EtherCAT via Lenze Controller x3xx/c32xx".

(2200003714, L-5589)

Remedy:

The error has been fixed. The i750 double axis can be added to the device list online again.

Type of publication:

Public (AKB on the Internet/Intranet)

### SM302-Safety parameter list: Error in plausibility check

Problem:

If "Special mode" is selected for "OMS: Function at low level" in the "Safety parameter list" for the Inverter 9400 with the SM302 safety module plugged in, an unnecessary error message "Incorrect parameterization for OMS via S-Bus!" appears and the safe parameter set cannot be loaded into the device.

(2200003499, L-4552)

Remedy:

The error has been fixed. There is no longer an error message, and the safe parameter set can be loaded into the device.

Type of publication:

Public (AKB on the Internet/Intranet)

### CSV format of the safe parameters differs from the format of the grey parameters

Problem:

In EASY Starter, both the safe parameters and the normal "grey" parameters can be exported to a csv file.

The format of the files differs from each other.

1. the table columns "Raw value" and "Raw value hex" are labelled differently.

2. the columns "Raw value" and "Raw value hex" are empty for the safe parameters.

(2200003409, L-4567)

Remedy:

Re 1: The names of the columns have been harmonized.

Re 2: As the values for "Raw value" and "Raw value hex" for the safe parameters are not available in the tool and cannot be calculated "safely", they can still not be exported.

Type of publication:

Public (AKB on the Internet/Intranet)

### Exception error when connecting via "Ethernet - EtherCAT to Lenze Controller c3xx/c32xx"

Problem:

The exception error for connection via "Ethernet - EtherCAT on Lenze Controller c3xx/c32xx" only occurs if an i750 double axis is connected to the EtherCAT of the Controller c3xx/c32xx.

(2200003714, L-5589)

Remedy:

The error has been fixed. A connection is possible without an exception error.

Type of publication:

Public (AKB on the Internet/Intranet)

### Error in plausibility check SM302 for OMS with "low level"

Problem:

If the special operation function is parameterized as a function for OMS at low level, a plausibility check error message is displayed and it is not possible to download the safe parameter set. (Both in the EASY Starter tool and in Engineer)

(2200003499, L-4552)

Remedy:

The error has only been fixed in the EASY Starter. There is no longer an error message and it is possible to download the safe parameter set.

As it has been decided not to create a new Engineer version, the error remains in Engineer V2.34.

Type of publication:

Public (AKB on the Internet/Intranet)

### Communication driver continues to run even though the EASY Starter has been stopped

Problem:

If the EASY Starter is stopped by a Beckhoff PLC in the Windows Task Manager during an online connection, the communication driver (LD2 service) remains in "Running" status.
After restarting the EASY Starter and stopping it again in the Task Manager, the EASY Starter no longer functions.

(L-4999)

Remedy:

The error has been fixed.

Type of publication:

Public (AKB on the Internet/Intranet)

### If several LDA files are opened at the same time, the Easy Starter only loads the first online device

Problem:

If several LDA files are opened at the same time, the Easy Starter only loads the first online device. The missing devices must be added individually to the online device list.

(L-5654)

Remedy:

The error has been fixed. 
If possible, an offline device and an online device linked to it are added to the device list 
for each lda file.

Type of publication:

Public (AKB on the Internet/Intranet)

### Installation may fail

Problem:

In a few cases, the installation of the EASY Starter may fail. 
The cause is a potential display name conflict with an existing Windows service "PackageManagerService" of another program (Avecto).

(2200004111, L-6894)

Remedy:

The error was fixed by renaming the Lenze Package Manager service.

Type of publication:

Public (AKB on the Internet/Intranet)

### Add 8400 motec only not possible over ADS EtherCAT

Problem:

An inverter 8400 motec cannot be added to the device list online when connected via ADS EtherCAT.

SDO error messages are displayed.

(L-5625)

Remedy:

The error was fixed.
An inverter 8400 motec can also be added to the device list online via ADS EtherCAT.

Type of publication:

Public (AKB on the Internet/Intranet)

## Product name - EASY Firmware Loader

| order Designation (type) | Number of the drawing (internal) | Brief description |
| --- | --- | --- |
| Available for download on the Internet as part of the EASY Starter Setup | Version 1.27 | EASY Firmware Loader 1.27 |

### 8400-Topline: Firmware cannot be loaded in boot mode

Problem:

No firmware can be loaded into an Inverter 8400-TopLine if the CPU of the inverter is in boot mode.

This can occur, for example, if there is no or faulty firmware in the device.

(L-5731)

Remedy:

The error has been fixed. The firmware can also be loaded in this case.

Type of publication:

Public (AKB on the Internet/Intranet)

### Exception after confirmation of an safety validation test

Problem:

An exception occurs and the tool hang when a safety validation test is confirmed by clicking "Test passed" or "Test failed".

This behavior occurred only if the inverter was enabled when the "Test passed" or "Test failed" button was pressed.

(2200003506, L-4763, L-5444, L-2582)

Remedy:

The error has been fixed.
There is no longer an exception, regardless of the status of the controller
(“activated” or “deactivated”).

Type of publication:

Public (AKB on the Internet/Intranet)

### Unhandled exception in case of invalid data in "Select Encoder" wizard

Problem:

If the user uses the “Select Encoder” wizard in i950 setting dialog “…\Motor feedback” it is possible to get an exception in the following steps.
(L-6022)

Remedy:

The error has been fixed.
If invalid entries are made, it is no longer possible to switch to the next step..

Type of publication:

Public (AKB on the Internet/Intranet)

# Compatibility with previous versions

The EASY Starter 1.27 including all included programs is functionally downwards compatible.

This means that V1.27 supports all functions of the previous version V1.26.