Acceptance protocol for i650 motec ES-IO device-replacement

This acceptance protocol is only designed for component replacement without change of the safety functionality.

General

| Date | 01.01.2026 |
| --- | --- |
| Carried out by |  |

Machine description

| Manufacturer |  |
| --- | --- |
| Machine / type / number |  |
| System / system part |  |
| Name of axis |  |

Execution

| Step | Description | Actual status / value |  |
| --- | --- | --- | --- |
| 1 | Switch on drive controller |  |  |
| 2 | Read serial number of drive controller (0x2000:002) or from housing |  |  |
| 3 | Read CRC of safe parameter set (0x2115:002) |  |  |
| If the index specified in step 3 can no longer be read out, use the values from the commissioning acceptance protocol for i650 motec ES-IO. |  |  |  |
| 4 | Read firmware version of ES-IO (0x212C:001) |  |  |
| 5 | Switch off drive controller |  |  |
| 6 | Replace device |  |  |
| 7 | Switch on drive controller |  |  |
| 8 | - Transfer application to drive controller - Reset the parameter set in the device and transfer safe parameter set to the drive controller |  |  |
| 9 | Read CRC of safe parameter set (0x2115:002) |  |  |
| 10 | Compare the CRC value with the value of step 3. |  |  |
| 11 | Are the CRC values identical? (This is the safety-relevant check!) | Please tick | yes |
|  |  |  | no |

Countersignature

The 'service engineer'

hereby confirms that the above-described tests and checks were carried out properly by skilled personnel.

| Date | Name | Company / department | Signature |
| --- | --- | --- | --- |